XIRCOM CREDITCARD ETHERNET ADAPTER FLASH UPDATE UTILITY

INSTALL DOCUMENT

Release 2.13 Revision E

December 16, 1993

=======================================================================

When CEUPDATE.EXE is running, power to the computer must NOT be

interrupted or the Xircom Adapter could be damaged.  We suggest

therefore, that the user does not rely on an internal battery alone.  Power

to the computer must be from a wall socket.

No PCMCIA Card Services drivers can be running at the same time as the

CEUPDATE.EXE.  The program does detect for these Card Services and will

exit out when a Card Service is detected.  The Card Service drivers will

need to be removed from the CONFIG.SYS or boot the system from a floppy

drive that has no CONFIG.SYS.  Once the card has been updated the Card

Service drivers can be restored.

Refer to the README.DOC that accompanies this file for other information

on CEUPDATE.EXE

Instructions:

The Flash Update Utility is designed to use only one memory window.

This memory window by default is at D000:0000 but can be user specified

by using the command line option "-m".

To invoke the flash update type:  CEUPDATE.EXE.  Messages will be displayed

from the update utility reporting success or failure.  CEUPDATE.EXE is the

only file needed to perform this update.

After using CEUPDATE.EXE, reboot the system.

To verify that the CEUPDATE.EXE utility upgraded the firmware to revision

'E', use CETEST.EXE.  It should display 'Rev. E' in the Model: field.

The utility does not function on the following computers:

=========================================================

Gateway 2000 Colerbook:  PCMCIA slot does not go to 12v.

Grid Convertible Pentop: PCMCIA slot does not go to 12v.

IBM 720 Thinkpad

===============================[  END  ]================================