�

���� ���� ������� ���� �  �    ���� ���� ����

�    �  � �  �  � �  � �  � �� �  � �  � �

���� ���� �  �  � ���� ����    ���� ���� ����

�                Consulting

Affordable, Quality Doorware for all BBS types.

1840 Haverhill Drive, Virginia Beach, VA 23456

BBS   (804) 468 - 1783 / 1492 / 1386

�����������������������������������������Ŀ

�                SatPro!                  �

� Copyright(c) 1995, Compu-Doc Consulting �

�           Written by Gary Smith         �

�������������������������������������������

Version 1.00R

Created By Gary R. Smith

Copyright (c) 1995 - Compu-Doc Consulting

License to Use Agreement

�����������������������������������������������������������������������

SatPro! is now shareware. We have spent hundreds of hours updating

and configuring SatPro! so that you can enjoy one of the ULTIMATE

satellite receive programs. This version is fully functional with

the exception that SatPro! will not write to disk any directed

EMAIL messages to your domain or operate in excess of 19200 baud

until registered.

CDC makes no warranties of any kind, expressed or implied, with

respect to the quality, performance, accuracy, or fitness of this

software product for any particular purpose.  CDC assumes no

responsibility for any decisions made or actions taken on the

part of the purchaser because of the use of the software

products.  In no event shall CDC, or any of its representatives,

be liable for any loss of profit or any other damages, arising

out of the use or inability to use this product - even if CDC or

an authorized agent of such has been advised of the possibility

of such damages - or for any claim by any other party.

CDC reserves the right to make changes, enhancements and

improvements to the software products at any time without

notice.

DISCLAIMER

���������������������������������������������������������������

There is made no warranty of any kind, express or implied

including, without limitation, any warranties of merchantability

and / or fitness for a particular purpose.  CDC shall not be

liable for any damages, whether direct, indirect, special or

sequential arising from a failure of this program to operate in

the manner desired by the User.  CDC shall not be liable for any

damage to data or property which may be caused directly or

indirectly by the User of this program.

IN NO EVENT WILL CDC BE LIABLE TO YOU FOR ANY DAMAGES, INCLUDING

ANY LOST PROFITS, LOST SAVINGS OR OTHER INCIDENTAL OR

CONSEQUENTIAL DAMAGES ARISING OUT OF YOUR USE OR INABILITY TO

USE THE PROGRAM, OR FOR ANY CLAIM BY ANY OTHER PARTY.

Borland C++ is a Registered Trademark of Borland International

SatPro!                                                       INTRODUCTION

��������������������������������������������������������������������������

SatPro! is a Communication Downlink software program that works with

satellite receivers to receive the compressed data packets produced by

PageSat Inc.

SatPro!                                                         SATCFG.EXE

��������������������������������������������������������������������������

Included with SatPro! is a program called SATCFG.EXE. This program was

designed to help you setup SatPro! with a minimum of trouble. There is

full context sensitive help available at the press of F1 during the

entire program.

To begin, Unzip the SatPro zip files into a directory (The examples used

through out this text will use C:\SATPRO as the sample directory. You

may place the file in any directory that you wish).

Now run the SATCFG program. This program will be currently unregistered

until you receive your registration number from Compudoc. However, this

program is FULLY FUNCTIONAL and will work fine without registration.

The next step is to start entering the data, First select:

1. COMM PORT CONFIGURATION

This is where you will define the comport status for SatPro!

Comm Port - Enter the Communications Port that you have your PageSat

Receiver attached to. (Comports must be from 1 - 16)

IRQ       - Enter the IRQ that you have the comport set to. Make sure

this is correct or you will lock up your computer when

SatPro! attempts to initialize the comport.

Baud Rate - Enter the baud rate appropriate for the feed that you

are receiving. Current Baud Rate for Pagesat is 19200.

SatPro! will not operate above this rate until registered.

Port Add  - Enter the address that your comport is set to. Examples

may be obtained on screen when you highlight the Port

address input area or you can get help by pressing F1.

<Once you have all the information entered press F10 to save it>

2. ACCEPTED FILES CONFIGURATION

In this area you will define which file types are to be accepted and

written to disk by SatPRO!

TYPE FIELD - This field is the type of file being transmitted by the

PageSat system. Current types include:

MAIL - EMAIL

NEWS - Internet News Feed

FIDO - FIDO backbone

STAT - Statistic file

RIME - Relay International Message Exchange (Coming?)

SUF FIELD - This field is the suffix that is used to identify file

types (Compression etc,...) The following types are

currently published by PageSat (Subject to change without

notice):

MAIL - BAG

NEWS - GZ

FIDO - XXX

RIME - XXX (Will be in the near future)

STAT - Text files for statistical information.

Note: The XXX files are files that are sent in batches. Or

files that have been split before sending them up to the

bird. The SatPro CONVERT will identify these file types

and build them once all parts have been sent.

PATH FIELD - Enter the FULL path to where your file types are to be

stored: (Ensure the directory exists or SatPRO! will

exit on the first file it attempts to write if the

directory does not exist!)

example: C:\PAGESAT\FIDO

C:\PAGESAT\MAIL etc...

3. FILE NAMES AND PATHS

Path for FERRORS.DAT

The file FERRORS.DAT is stored in the directory specified. This file

contains information on files that were corrupted enroute to your

system. They messages are in the format of:

TIME     TYPE FORM FILENAME         SIZE   REASON

19:22:55 NEWS GZIP pri950522-161123 039221 (Aborted) foot print not

found

Path for Statistics File (STATxxxx.STA)

The file STATxxxx.STA will be written to the path specified. This is

the nightly statistics file that is sent for comparrison from

PageSat. (Note: xxxx is the sequence number that the file was written

in)

4. SYSTEM INFORMATION

Registration Number - Enter your registration number as received

from Compudoc! If you do not have a registration number you may

obtain one easily. See the ORDER.FRM located in the SatPro! zip file.

Registration Name - Enter your registration name as registered

with Compudoc! If you do not have a registration name you may obtain

one easily. See the ORDER.FRM located in the SatPro! zip file.

Your systems Internet Domain Address - SatPro! will use this

address to seperate and store your email if you have MAIL selected

in the Accepted Files Area (Available in registered version only)

Example: compudoc.com\p

Time Slices - There are various positions within SatPro! that

time slices (1/18 of a second) may be given up. There are several

areas that by default can give up several at a time. These areas

are the places that SatPro! waits for the receiver to build up the

buffer. The maximum size of the buffer can reach 16k. If you find

SatPro! locking up it is probably because you have to many time

slices being given and it is overwriting the inbound file buffer.

Recommended Settings:

DOS                0

WIN, OS2, WIN/NT   4-6

DesqView           1-2

These may not be ideal for your system, Try these to start and

move up or down from there.

File Counter - This is the file counter number that SatPro! last

named a file. The file counter will pickup from this number. Its

recommended that this number not be changed so that files will not

be written over.

SatPro!                                                         SATPRO.EXE

��������������������������������������������������������������������������

To Start SatPro! there is one required command line option, this is the

path to where SatPro is located. There are two files that SatPro! needs

to access (COMM.DAT, FILEDAT.DAT).

Example:  SatPro C:\Satpro

SatPro! will automatically detect if you are using 16550 style Uarts

and will trigger the FIFO automatically. Hardware flow control will

also be used if available.

What you will see on the SatPro! screen:

SatPro! provides you with the most information available at the present

time. Below is a description of each item:

INCOMING FILE INFORMATION

file name - This is the name of the current file being downloaded.

If no file is in the que then this field is blank.

file type - This is the type of file being sent from PageSat. It

can be of different types. Types currently known are

FIDO (Fido backbone) NEWS (Internet News Groups)

MAIL (Domain Email) STAT (Statistics File).

file size / - This field contains two numeric values, The left value

is the expected size of the current file, the right

value is the size of the file in the buffer. When the

files are complete these files are written to disk.

PREVIOUS FILE INFORMATION

file name - This is the name of the last file downloaded.

file type - This was the type of file sent from PageSat.

file size - This is the size of the last file that was written

to disk.

placed in - This is the directory that the file was placed in.

If the last file was EMAIL that was being routed to

another domain, SatPro! will enter "Forwarded to

another Domain!"

LAST ERROR MESSAGE

If there was a problem with the last file received, or if it is

a type of file not specified as an ACCEPTED FILE TYPE then that

error message will be written to this space. If there is no

error for the last file then this space will be blank.

SYSTEM INFORMATION

flow control - If SatPro! can detect and use flow control then

this field will say YES else it will enter NO.

16550 uart   - If SatPro! can detect and use a 16550 uart then this

field will say YES else it will enter a NO value. It

is recommended that you use 16550 uarts because of

the buffer value. 16450 do not contain a buffer and

can be the cause of overruns.

detected     - On the right side of this area SatPro! will indicate

what type of system it detected. SatPro! uses this

information to correctly distribute time slices under

the current operating system.

SatPro!                                                           ADDENDUM

��������������������������������������������������������������������������

File Logging

A file SatPro.STA is now created in the directory that you specified

in the SFCFG program. It will include startup and shutdown times and

file information that is currently in the same format as the Pagesat

statistics file that is sent out around midnight PST.

There is also a FERROR.DAT file created in the directory specified. This

file contains files that were not intercepted due to overuns and such.

This file will be used at a later date to retrieve dupe files. You may

delete it nightly.

END OF DOCUMENTATION