ASPI for Win32

691 South Milpitas Blvd.

Milpitas, CA  95035

This page intentionally left blank

Preface

COPYRIGHT

No part of this publication may be reproduced, stored in a retrieval system, or transmitted, in any form or by any means, electronic, mechanical, photocopying, recording, or otherwise, without the prior written consent of Adaptec, Inc.  691 South Milpitas Blvd., Milpitas, CA  95035 and Microsoft Corp. One Microsoft Way, Redmond, WA 98052-6399

CHANGES

The material in this manual is for information only and is subject to change without notice.

While reasonable efforts have been taken in the preparation of this manual to assure its accuracy, Adaptec, Inc. assumes no liability resulting from any errors or omissions in this manual, or from the use of the information contained herein.

Adaptec reserves the right to make changes in the product design without reservation and without notification to its users.

Additional information may be obtained from:

691 South Milpitas Blvd.

Milpitas, CA  95035

This page intentionally left blank

Introduction

The Advanced SCSI Programming Interface (ASPI) has become an industry standard SCSI software interface in the DOS, Windows, OS/2, and Netware environments. ASPI allows developers to write device drivers and applications which are independent of the specific host adapter platform used to connect SCSI peripherals. With ASPI, there is a single driver managing the host bus adapter, eliminating conflicts when multiple applications try to access the SCSI bus at the same time.

With the arrival of Windows NT and Windows 95, ASPI must be extended to allow applications to take full advantage of these new, 32-bit operating systems. This specification describes the Advanced SCSI Programming Interface (ASPI) for the Win32 environment.

Note: This specification defines the protocol between ASPI client applications and the ASPI Manager wnaspi32.dll. The specification does not describe the protocol between wnaspi32.dll and the target Win32 environment, since this implementation may vary between operating systems.

Definitions

ASPI:		Advanced SCSI Programming Interface.

SRB:			SCSI Request Block:  Used by both drivers or application programs to send commands to the ASPI manager.

SCSI:		Small Computer Systems Interface.

DLL:			Dynamic Link Library.

ASPI Client:	An application or driver that uses ASPI to access SCSI devices.

ASPI Manager:	The service layer which processes ASPI requests.

Using ASPI for Win32 - An Overview

The ASPI manager for Win32 is implemented as a Dynamic Link Library (DLL) called wnaspi32.dll. ASPI function calls can be used to retrieve information about installed SCSI host adapters and devices, and to execute SCSI I/O requests. The ASPI for Win32 layer is fully re-entrant, supporting multitasking and multithreaded operations. To use ASPI for Win32, the following two functions must be imported from wnaspi32.dll into the client's Win32 application.

| Function | Description |
| --- | --- |
| GetASPI32SupportInfo | This function returns the number of SCSI host adapters installed and ensures that the ASPI manager is initialized properly. Clients must call this function once at initialization time before the SendASPI32Command function can be accessed. |
| SendASPI32Command | This function handles requests for SCSI I/O. |

The following SCSI I/O command codes are defined within the SendASPI32Command:

| Value | Meaning |
| --- | --- |
| SC_HA_INQUIRY | Get information about installed host adapters, including the number of installed adapters. |
| SC_GET_DEV_TYPE | Get information about installed SCSI devices. |
| SC_EXEC_SCSI_CMD | Execute SCSI I/O. |
| SC_ABORT_SRB | Abort an outstanding I/O request. |
| SC_RESET_DEV | Reset an individual SCSI target. |
| SC_GET_DISK_INFO | Get information on disk type SCSI devices. (Not used with Windows NT) |

GetASPI32SupportInfo Function

Prototype: DWORD GetASPI32SupportInfo(VOID)

The GetASPI32SupportInfo function returns the number of host adapters installed and ensures that the ASPI manager is initialized properly. This function must be called once at initialization time, before SendASPI32Command is accessed. The number of host adapters returned represents the logical bus count, not the true physical adapter count. For host adapters with a single bus, the host adapter count and logical bus count are identical.

This function has no parameters.

The DWORD return value specifies the result of the ASPI request. The DWORD is encoded as follows:

Table 4-1. Return Values from GetASPI32SupportInfo Function

| Length | Bit | Meaning |  |
| --- | --- | --- | --- |
| WORD | 31-16 | Reserved = 0 |  |
| BYTE | 15-8 | Status |  |
|  |  | SS_COMP | ASPI request completed without error |
|  |  | SS_FAILED_INIT | ASPI manager unable to initialize; ASPI services not available |
| BYTE | 7-0 | If Status = SS_COMP | Number of host adapters |

Example

This example returns the current status of ASPI for Win32.

DWORD ASPIStatus;

BYTE NumAdapters;

HWND hwnd;

.

.

ASPIStatus = GetASPI32SupportInfo();

switch ( HIBYTE(LOWORD(ASPIStatus)) )

{

case SS_COMP:

/*

* ASPI for Win32 is properly initialized

*/

NumAdapters = LOBYTE(LOWORD(ASPIStatus));

break;

default:

MessageBox( hwnd,

"ASPI for Win32 is not initialized!!",

NULL,

MB_ICONSTOP );

return FALSE;

}

.

.

SendASPI32Command Function

Prototype: DWORD SendASPI32Command(LPSRB)

Declaration: LPSRB lpSRB;

The SendASPI32Command function handles all SCSI I/O requests. A command code is used to specify the type of I/O requested. SendASPI32Command is called with a pointer to a SCSI Request Block (SRB) structure. While SRB definition can vary depending on the ASPI command code, all SRBs include a standard SRB header:

typedef struct {

BYTE		SRB_Cmd;				// ASPI command code

BYTE		SRB_Status;			// ASPI command status byte

BYTE		SRB_HaId;				// ASPI host adapter number

BYTE		SRB_Flags;			// ASPI request flags

DWORD		SRB_Hdr_Rsvd;		// Reserved, MUST = 0

} SRB_Header;

The SRB_Cmd field must contain the command code for the desired SCSI I/O operation. Each of these commands is described in the following sections. Possible values for SRB_Cmd include the following:

SC_HA_INQUIRY
SC_GET_DEV_TYPE
SC_EXEC_SCSI_CMD
SC_ABORT_SRB
SC_RESET_DEV
SC_GET_DISK_INFO

The SC_GET_DISK_INFO command is not used with Windows NT.

Win32 ASPI Commands

Note: In the following sections, fields marked with [OUT] are returned by the ASPI manager. Fields marked with [IN] are set by the ASPI client before they are sent to the ASPI manager. Fields marked as Reserved must be zeroed before the SRB is sent to the ASPI manager.

Host Adapter Inquiry Command

Prototype: DWORD SendASPI32Command(LPSRB)

Declaration: LPSRB lpSRB;

The SendASPI32Command function with command code SC_HA_INQUIRY is used to get information on the installed host adapter hardware, including the number of host adapters installed. The number of host adapters returned represents the logical bus count instead of the true physical adapter count. For host adapters that support single bus only, the host adapter count and logical bus count are identical. For host adapters that support multiple buses, the host adapter count represents the total logical bus count.

| Parameter | Description |
| --- | --- |
| lpSRB | Points to the following SCSI request block |

typedef struct {

BYTE		SRB_Cmd;					// ASPI command code = SC_HA_INQUIRY

BYTE		SRB_Status;				// ASPI command status byte

BYTE		SRB_HaId;					// ASPI host adapter number

BYTE		SRB_Flags;				// ASPI request flags

DWORD		SRB_Hdr_Rsvd;			// Reserved, MUST = 0

BYTE		HA_Count;					// Number of host adapters present

BYTE		HA_SCSI_ID;				// SCSI ID of host adapter

BYTE		HA_ManagerId[16];		// String describing the manager

BYTE		HA_Identifier[16];		// String describing the host adapter

BYTE		HA_Unique[16];			// Host Adapter Unique parameters

WORD		HA_Rsvd1;

} SRB_HAInquiry, *PSRB_HAInquiry;

Table 4-2. Host Adapter Inquiry Command

| Member | Description |  |
| --- | --- | --- |
| SRB_Cmd | [IN] This field must contain SC_HA_INQUIRY. |  |
| SRB_Status | [OUT] On return, this field is the same as the return status defined below. |  |
| SRB_HaId | [IN] This field specifies which installed host adapter the request is intended for. Host adapter numbers are always assigned by the SCSI manager layer, beginning with zero. |  |
| SRB_Flags | [IN] Reserved = 0 |  |
| SRB_Hdr_Rsvd | Reserved = 0 |  |
| HA_Count | [OUT] The ASPI manager sets this field with the number of host adapters installed under ASPI. For example, a return value of 2 indicates that host adapters #0 and #1 are valid. To determine the total number of host adapters in the system, the SRB_HaId field should be set to zero, or GetASPI32SupportInfo can be used. The number of host adapters returned represents the logical bus count instead of the true physical adapter count. For host adapters that support single bus only, the host adapter count and logical bus count are identical. |  |
| HA_SCSI_ID | [OUT] The ASPI manager sets this field with the SCSI ID of the given host adapter. |  |
| HA_ManagerId[..] | [OUT] The ASPI manager fills this 16-character buffer with an ASCII string describing the ASPI manager. For ASPI for WIN32, the string “ASPI for WIN32“ is returned. |  |
| HA_Identifier[..] | [OUT] The ASPI manager fills this 16-character buffer with a string describing the host adapter. |  |
| HA_Unique[..] | [OUT] The ASPI manager fills this 16-byte buffer with host adapter unique parameters. The returned buffer is encoded as follows: |  |
|  | Byte 15-8 | Reserved = 0 |
|  | Byte 7-4 | Maximum Transfer Length. |
|  | Byte 3 | Maximum SCSI Targets. Indicates the maximum number of targets (SCSI IDs) the adapter supports. If this value is not set, it is assumed that there are 8 targets. (SCSI IDs 0-7). |
|  | Byte 2 | Adapter Unique Flags: |
|  |  | Bit 7-2 Reserved Bit 1 1 = Residual byte count reporting is supported 0 = Residual byte count reporting is not supported. See Residual Byte Length on page 6 for more information. Bit 0 Reserved |
|  | Byte 1-0 | Buffer Alignment Mask. The host adapter requires data buffer alignment specified by the 16-bit value. A value of 0x0000 indicates no boundary requirements (e.g. byte), 0x0001 word alignment, 0x0003 double-word, 0x0007 8-byte alignment, etc. The 16-bit value allows data buffer alignments of up to 65536-byte boundaries. Byte 1 is the most significant byte. |

Table 4-3. Return Values from Host Adapter Inquiry Command

| Value | Meaning |
| --- | --- |
| SS_COMP | SCSI/ASPI request has completed without error. |
| SS_INVALID_HA | Invalid host adapter number. |

Residual Byte Length

Residual byte length is the number of bytes not transferred to, or received from, the target SCSI device. For example, if the ASPI buffer length for a SCSI Inquiry command is set for 100 bytes, but the target only returns 36 bytes; the residual length is 64 bytes. If the ASPI buffer length for a SCSI Write command is set for 514 bytes but the target only takes 512 bytes, the residual length is 2 bytes. ASPI modules can determine if the loaded ASPI manager supports residual byte length by issuing an Extended Host Adapter Inquiry command, as described in the previous section.

Example

This example of the Host Adapter Inquiry command gets host adapter hardware information from adapter #0.

SRB_HAInquiry MySRB;

DWORD ASPIStatus;

.

.

MySRB.SRB_Cmd 				= SC_HA_INQUIRY;

MySRB.SRB_HaId				= 0;

MySRB.SRB_Flags				= 0;

MySRB.SRB_Hdr_Rsvd			= 0;

ASPIStatus					= SendASPI32Command ( (LPSRB) &MySRB );

.

.

Get Device Type Command

Prototype: DWORD SendASPI32Command(LPSRB)

Declaration: LPSRB lpSRB;

The SendASPI32Command function with command code SC_GET_DEV_TYPE enables you to identify the devices available on the SCSI bus. A Win32 tape backup package, for example, can scan each target/LUN on each installed host adapter looking for a device type corresponding to sequential access devices. This eliminates the need for each Win32 application to duplicate the effort of scanning the SCSI bus for devices.

| Parameter | Description |
| --- | --- |
| lpSRB | Points to the following SCSI request block |

typedef struct {

BYTE		SRB_Cmd;				// ASPI command code = SC_GET_DEV_TYPE

BYTE		SRB_Status;			// ASPI command status byte

BYTE		SRB_HaId;				// ASPI host adapter number

BYTE		SRB_Flags;			// Reserved

DWORD		SRB_Hdr_Rsvd;		// Reserved

BYTE		SRB_Target;			// Target's SCSI ID

BYTE		SRB_Lun;				// Target's LUN number

BYTE		SRB_DeviceType;		// Target's peripheral device type

BYTE		SRB_Rsvd1;			// Reserved for alignment

} SRB_GDEVBlock, *PSRB_GDEVBlock;

Table 4-4. Get Device Type Command

| Member | Description |
| --- | --- |
| SRB_Cmd | [IN] This field must contain SC_GET_DEV_TYPE. |
| SRB_Status | [OUT] On return, this field will be the same as the return status defined below. |
| SRB_HaId | [IN] This field specifies which installed host adapter the request is intended for. Host adapter numbers are always assigned by the ASPI manager layer beginning with zero. |
| SRB_Flags | [IN] Reserved = 0 |
| SRB_Hdr_Rsvd | Reserved = 0 |
| SRB_Target | [IN] SCSI ID of target device. |
| SRB_Lun | [IN] Logical Unit Number (LUN) of device. |
| SRB_DeviceType | [OUT] The ASPI manager fills this field with the peripheral device type. This is derived from the Peripheral Device Type returned by the SCSI Inquiry command. Refer to the SCSI specification to learn more about the SCSI Inquiry command. |

Table 4-5. Return Values from Get Device Type Command

| Value | Meaning |
| --- | --- |
| SS_COMP | SCSI/ASPI request has completed without error. |
| SS_INVALID_HA | Invalid host adapter number. |
| SS_NO_DEVICE | SCSI device not installed. |

Example

This example gets the peripheral device type from host adapter #0, target ID #4, and LUN #0.

SRB_GDEVBlock MySRB;

DWORD ASPIStatus;

.

.

MySRB.SRB_Cmd 			= SC_GET_DEV_TYPE;

MySRB.SRB_HaId			= 0;

MySRB.SRB_Flags			= 0;

MySRB.SRB_Hdr_Rsvd		= 0;

MySRB.SRB_Target			= 4;

MySRB.SRB_Lun			= 0;

ASPIStatus				= SendASPI32Command ( (LPSRB) &MySRB );

.

/****************************************************************/

/* If MySRB.SRB_Status == SS_COMP, MySRB.SRB_DeviceType

/* will contain the peripheral device type.

/****************************************************************/

.

.

Execute SCSI I/O Command

Prototype: DWORD SendASPI32Command(lpSRB)

Declaration: LPSRB lpSRB;

The SendASPI32Command function with command code SC_EXEC_SCSI_CMD is used to execute a SCSI I/O command. Once an ASPI client has initialized, virtually all I/O is performed with this command.

| Parameter | Description |
| --- | --- |
| lpSRB | Points to the following SCSI request block |

typedef struct {

BYTE		SRB_Cmd;						// ASPI command code = SC_EXEC_SCSI_CMD

BYTE		SRB_Status;					// ASPI command status byte

BYTE		SRB_HaId;						// ASPI host adapter number

BYTE		SRB_Flags;					// ASPI request flags

DWORD		SRB_Hdr_Rsvd;				// Reserved

BYTE		SRB_Target;					// Target's SCSI ID

BYTE		SRB_Lun;						// Target's LUN number

WORD		SRB_Rsvd1;					// Reserved for Alignment

DWORD		SRB_BufLen;					// Data Allocation Length

BYTE		*SRB_BufPointer;				// Data Buffer Point

BYTE		SRB_SenseLen;				// Sense Allocation Length

BYTE		SRB_CDBLen;					// CDB Length

BYTE		SRB_HaStat;					// Host Adapter Status

BYTE		SRB_TargStat;				// Target Status

void		(*SRB_PostProc)();			// Post routine

void		*SRB_Rsvd2;					// Reserved

BYTE		SRB_Rsvd3[16];				// Reserved for expansion

BYTE		CDBByte[16];					// SCSI CDB

BYTE		SenseArea[SENSE_LEN+2];	// Request Sense buffer

} SRB_ExecSCSICmd, *PSRB_ExecSCSICmd;

Table 4-6. Execute SCSI I/O Command

| Member | Description |  |  |
| --- | --- | --- | --- |
| SRB_Cmd | [IN] This field must contain SC_EXEC_SCSI_CMD. |  |  |
| SRB_Status | [OUT]This field returns one of the following statuses: |  |  |
|  | SS_PENDING | SCSI request is in progress. |  |
|  | SS_COMP | SCSI/ASPI request has completed without error. |  |
|  | SS_ABORTED | SCSI command has been aborted. |  |
|  | SS_ABORT_FAIL | SCSI command abort failed. |  |
|  | SS_ERR | SCSI command completed with an error. |  |
|  | SS_INVALID_SRB | One or more parameters in the SCSI Request Block (SRB) are set incorrectly. |  |
|  | SS_INVALID_PATH_ID | SCSI ID and LUN are invalid. |  |
|  | SS_BUFFER_TO_BIG | The ASPI manager cannot handle the given transfer size. See Programming Guidelines on page 27 for more information. |  |
|  | SS_BUFFER_ALIGN | The ASPI manager cannot handle the alignment on this buffer. You must force the buffer into the alignment specified by the alignment mask returned as part of the SC_HA_INQUIRY command. |  |
|  | SS_SECURITY_VIOLATION | The application does not have the security privileges needed to perform the operation for the device. |  |
| SRB_HaId | [IN] This field specifies which installed host adapter the request is intended for. Host adapter numbers are always assigned by the SCSI manager layer beginning with zero. |  |  |
| SRB_Flags | [IN] The SRB flags field is defined as follows: |  |  |
|  | SRB_DIR_IN | Data transfer from SCSI target to host. Mutually exclusive with SRB_DIR_OUT. |  |
|  | SRB_DIR_OUT | Transfer from host to SCSI target. Mutually exclusive with SRB_DIR_IN. |  |
|  | SRB_EVENT_NOTIFY | Enable ASPI command event notification. See Request Completion/Notification on page 22 for more details. This flag and the SRB_POSTING flag are mutually exclusive. |  |
|  | SRB_POSTING | Enable ASPI command completion posting. See Request Completion/Notification on page 22 for more details. This flag and the SRB_EVENT_NOTIFY flag are mutually exclusive. |  |
|  | SRB_ENABLE_RESIDUAL COUNT | Enables reporting of residual byte count. This bit is only significant if the host adapter reports support for residual byte count from the SC_HA_INQUIRY command. Whenever a data underrun occurs, the SRB_BufLen field is updated to reflect the remaining bytes to transfer. |  |
| SRB_Hdr_Rsvd | Reserved = 0 |  |  |
| SRB_Target | [IN] SCSI ID of target device. |  |  |
| SRB_Lun | [IN] Logical Unit Number (LUN) of device. |  |  |
| SRB_BufLen | [IN] This field indicates the number of bytes to be transferred. If the SCSI command to be executed does not transfer data (e.g., Test Unit Ready, Rewind, etc.), this field must be set to zero. If residual byte length is supported and selected, this field is returned with the residual number of bytes (usually 0). See Residual Byte Length on page 6. |  |  |
| SRB_BufPointer | [IN] This field is a pointer to the data buffer. |  |  |
| SRB_SenseLen | [IN] This field indicates the number of bytes allocated at the end of the SRB for sense data. A request sense is automatically generated if a check condition is presented at the end of a SCSI command. |  |  |
| SRB_CDBLen | [IN] This field establishes the length, in bytes, of the SCSI Command Descriptor Block (CDB). This value is typically 6 or 10. |  |  |
| SRB_HaStat | [OUT] Upon completion of the SCSI command, the ASPI manager sets this field with the host adapter status as follows: |  |  |
|  | HASTAT_OK |  | Host adapter did not detect an error. |
|  | HASTAT_TIMEOUT |  | The time allocated for the transaction ran out. |
|  | HASTAT_COMMAND_TIMEOUT |  | SRB expired while waiting to be processed. |
|  | HASTAT_SEL_TO |  | Selection of target timed out. |
|  | HASTAT_MESSAGE_REJECT |  | While processing SRB, the adapter received a MESSAGE REJECT. |
|  | HASTAT_BUS_RESET |  | A bus reset was detected. |
|  | HASTAT_PARITY_ERROR |  | A parity error was detected. |
|  | HASTAT_REQUEST_SENSE_FAILED |  | The adapter failed in issuing a Request Sense after a check condition was reported by the target device. |
|  | HASTAT_DO_DU |  | Data overrun/underrun. |
|  | HASTAT_BUS_FREE |  | Unexpected Bus Free. |
|  | HASTAT_PHASE_ERR |  | Target Bus phase sequence failure. |
| SRB_TargStat | [OUT] Upon completion of the SCSI command, the ASPI manager will record the specific status returned by the SCSI target: |  |  |
|  | STATUS_GOOD |  | No target status. |
|  | STATUS_CHKCOND |  | Check status (sense data is in SenseArea). |
|  | STATUS_BUSY |  | Specified Target/LUN is busy. |
|  | STATUS_RESCONF |  | Reservation conflict. |
| SRB_PostProc | [IN] If posting is enabled (SRB_POSTING flag), this field contains a pointer to a function. ASPI for Win32 calls this function upon completion of an ASPI request. If event notification is enabled (SRB_EVENT_NOTIFY flag), this field contains a handle to an event. ASPI for Win32 signals this event upon completion of an ASPI request. See Request Completion/Notification on page 22. |  |  |
| CDBByte[..] | [IN] This field contains the CDB as defined by the target's SCSI command set. The length of the SCSI CDB is specified in the SRB_CDBLen field. |  |  |
| SenseArea[..] | [OUT] The SenseArea is filled with the sense data on a check condition. The maximum length of this field is specified in the SRB_SenseLen field. Note that the target can return fewer than the number of sense bytes requested. |  |  |

Table 4-7. Return Values from Execute SCSI I/O Command

| Value | Meaning |
| --- | --- |
| SS_PENDING | SCSI request is in progress. |

Example

This example sends a SCSI Inquiry command to host adapter #0, target #0, LUN #0.

SRB_ExecSCSICmd MySRB;

DWORD ASPIStatus;

char InquiryBuffer[32];

.

.

.

MySRB.SRB_Header				= SC_EXEC_SCSI_CMD;

MySRB.SRB_HaId				= 0;

MySRB.SRB_Flags				= SRB_DIR_IN | SRB_POSTING;

MySRB.SRB_Hdr_Rsvd			= 0;

MySRB.SRB_Target				= 0;

MySRB.SRB_Lun				= 0;

MySRB.SRB_BufLen				= 32;

MySRB.SRB_SenseLen			= SENSE_LEN;

MySRB.SRB_BufPointer		= InquiryBuffer;

MySRB.SRB_CDBLen				= 6;

MySRB.RB_PostProc			= PostProcedure;

MySRB.CDBByte[0]				= SCSI_INQUIRY;

MySRB.CDBByte[1]				= 0;

MySRB.CDBByte[2]				= 0;

MySRB.CDBByte[3]				= 0;

MySRB.CDBByte[4]				= 32;

MySRB.CDBByte[5]				= 0;

.

/***************************************************/

/* Make sure all other reserved fields are zeroed

/* before passing the SRB to ASPI for Win32

/***************************************************/

.

ASPIStatus = SendASPI32Command ( (LPSRB) &MySRB );

.

.

Abort SCSI I/O Command

Prototype: DWORD SendASPI32Command(LPSRB)

Declaration: LPSRB lpSRB;

The SendASPI32Command function with command code SC_ABORT_SRB is used to request that a pending SRB be aborted. It should be issued on any I/O request that has not completed if the application wishes to time-out on that request. Success of the abort command is never assured.

| Parameter | Description |
| --- | --- |
| lpSRB | Points to the following SCSI request block |

typedef struct {

BYTE		SRB_Cmd;				// ASPI command code = SC_ABORT_SRB

BYTE		SRB_Status;			// ASPI command status byte

BYTE		SRB_HaId;				// ASPI host adapter number

BYTE		SRB_Flags;			// Reserved

DWORD		SRB_Hdr_Rsvd;		// Reserved

void		*SRB_ToAbort;		// Pointer to SRB to abort

} SRB_Abort, *PSRB_Abort;

Table 4-8. Abort SCSI I/O Command

| Member | Description |
| --- | --- |
| SRB_Cmd | [IN] This field must contain SC_ABORT_SRB. |
| SRB_Status | [OUT] On return, this field will be the same as the return status defined below. |
| SRB_HaId | [IN] This field specifies which installed host adapter the request is intended for. Host adapter numbers are always assigned by the ASPI manager layer beginning with zero. |
| SRB_Flags | [IN] Reserved = 0 |
| SRB_Hdr_Rsvd | Reserved = 0 |
| SRB_ToAbort | [IN] This fields contains a pointer to the SRB which is to be aborted. The actual failure or success of the abort operation is indicated by the status eventually returned in this SRB. |

Table 4-9. Return Values from Abort SCSI I/O Command

| Value | Meaning |
| --- | --- |
| SS_COMP | SCSI/ASPI request has completed without error. |
| SS_INVALID_HA | Invalid host adapter number. |
| SS_INVALID_SRB | One or more parameters in the SCSI Request Block (SRB) are set incorrectly. |

Example

This example shows how to abort a "stuck" SCSI I/O.

SRB_ExecSCSICmd StuckSRB;

SRB_Abort AbortSRB;

DWORD ASPIStatus;

.

.

AbortSRB.SRB_Cmd 			= SC_ABORT_SRB;

AbortSRB.SRB_HaId			= 0;

AbortSRB.SRB_Flags			= 0;

AbortSRB.SRB_Hdr_Rsvd		= 0;

AbortSRB.SRB_ToAbort		= (LPSRB) &StuckSRB;

ASPIStatus 					= SendASPI32Command ( (LPSRB) &AbortSRB );

.

.

Reset SCSI Device Command

Prototype: DWORD SendASPI32Command(LPSRB)

Declaration: LPSRB lpSRB;

The SendASPI32Command function with command code SC_RESET_DEV is used to send a SCSI Bus Device reset to the specified target.

| Parameter | Description |
| --- | --- |
| lpSRB | Points to the following SCSI request block |

typedef struct {

BYTE		SRB_Cmd;				// ASPI command code = SC_RESET_DEV

BYTE		SRB_Status;			// ASPI command status byte

BYTE		SRB_HaId;				// ASPI host adapter number

BYTE		SRB_Flags;			// Reserved

DWORD		SRB_Hdr_Rsvd;		// Reserved

BYTE		SRB_Target;			// Target's SCSI ID

BYTE		SRB_Lun;				// Target's LUN number

BYTE		SRB_Rsvd1[12];		// Reserved for Alignment

BYTE		SRB_HaStat;			// Host Adapter Status

BYTE		SRB_TargStat;		// Target Status

void		*SRB_PostProc;		// Post routine

void		*SRB_Rsvd2;			// Reserved

BYTE		SRB_Rsvd3[32];		// Reserved

} SRB_BusDeviceReset, *PSRB_BusDeviceReset;

Table 4-10. Reset SCSI Device Command

| Member | Description |  |
| --- | --- | --- |
| SRB_Cmd | [IN] This field must contain SC_RESET_DEV. |  |
| SRB_Status | [OUT] This field returns one of the following status: |  |
|  | SS_PENDING | SCSI request is in progress. |
|  | SS_COMP | SCSI/ASPI request has completed without error. |
|  | SS_ABORTED | SCSI command has been aborted. |
|  | SS_ABORT_FAIL | SCSI command abort failed. |
|  | SS_ERR | SCSI command completed with an error. |
|  | SS_INVALID_SRB | One or more parameters in the SCSI Request Block (SRB) are set incorrectly. |
|  | SS_INVALID_PATH_ID | SCSI ID and LUN are invalid. |
| SRB_HaId | [IN] This field specifies which installed host adapter the request is intended for. Host adapter numbers are always assigned by the SCSI manager layer beginning with zero. |  |
| SRB_Flags | [IN] Reserved = 0 |  |
| SRB_Hdr_Rsvd | Reserved = 0 |  |
| SRB_Target | [IN] SCSI ID of target device. |  |
| SRB_Lun | [IN] Logical Unit Number (LUN) of device. This field is ignored by ASPI for Win32, since SCSI Bus Device resets are done on a per-target basis only. |  |
| SRB_HaStat | [OUT] Upon completion of the SCSI command, the ASPI manager sets this field with the host adapter status to one of the following values: |  |
|  | HASTAT_OK | Host adapter did not detect an error. |
|  | HASTAT_TIMEOUT | The time allocated for the operation expired. |
|  | HASTAT_COMMAND_TIMEOUT | SRB expired while waiting to be processed. |
|  | HASTAT_SEL_TO | Selection of target timed out. |
|  | HASTAT_MESSAGE_REJECT | While processing SRB, the adapter received a MESSAGE REJECT. |
|  | HASTAT_BUS_RESET | A bus reset was detected. |
|  | HASTAT_PARITY_ERROR | A parity error was detected. |
|  | HASTAT_REQUEST_SENSE_FAILED | The adapter failed in issuing a request sense after a check condition was reported by the target device. |
|  | HASTAT_DO_DU | Data overrun/underrun. |
|  | HASTAT_BUS_FREE | Unexpected Bus Free. |
|  | HASTAT_PHASE_ERR | Target Bus phase sequence failure. |
| SRB_TargStat | [OUT] Upon completion of the SCSI command, the ASPI manager sets this field with the target status as follows: |  |
|  | STATUS_GOOD | No target status. |
|  | STATUS_CHKCOND | Check status (sense data is in SenseArea). |
|  | STATUS_BUSY | Specified Target/LUN is busy. |
|  | STATUS_RESCONF | Reservation conflict. |
| SRB_PostProc | [IN] If posting is enabled, ASPI for Win32 will post completion of an ASPI request to this function pointer. See Request Completion/Notification on page 22. |  |

Table 4-11. Return Values from Reset SCSI Device Command

| Value | Meaning |
| --- | --- |
| SS_PENDING | SCSI request is in progress. |

Example

This example issues a SCSI bus device reset to host adapter #0, target #5.

SRB_BusDeviceReset MySRB;

DWORD ASPIStatus;

.

.

MySRB.SRB_Header			= SC_RESET_DEV;

MySRB.SRB_HaId			= 0;

MySRB.SRB_Flags			= 0;

MySRB.SRB_Hdr_Rsvd		= 0;

MySRB.SRB_Target			= 5;

MySRB.SRB_Lun			= 0;

ASPIStatus 				= SendASPI32Command ( (LPSRB) &MySRB );

.

.

Get SCSI Disk Information Command

Prototype: DWORD SendASPI32Command(LPSRB)

Declaration: LPSRB lpSRB;

The SendASPI32Command function with command code SC_GET_DISK_INFO is used to obtain information about a disk type SCSI device. The information returned includes BIOS Int 13h control and accessibility of the device, the drive’s Int 13h physical drive number, and the geometry used by the Int 13h services for the drive.

Note: This command is not valid for Windows NT, which does not use the Int 13 interface.

| Parameter | Description |
| --- | --- |
| lpSRB | Points to the following SCSI request block |

typedef struct {

BYTE		SRB_Cmd;					// ASPI command code = SC_EXEC_SCSI_CMD

BYTE		SRB_Status;				// ASPI command status byte

BYTE		SRB_HaId;					// ASPI host adapter number

BYTE		SRB_Flags;				// Reserved

DWORD		SRB_Hdr_Rsvd;			// Reserved

BYTE		SRB_Target;				// Target's SCSI ID

BYTE		SRB_Lun;					// Target's LUN number

BYTE		SRB_DriveFlags;			// Driver flags

BYTE		SRB_Int13HDriveInfo;	// Host Adapter Status

BYTE		SRB_Heads;				// Preferred number of heads translation

BYTE		SRB_Sectors;				// Preferred number of sectors translation

BYTE		SRB_Rsvd1[10];			// Reserved

} SRB_GetDiskInfo, *PSRB_GetDiskInfo;

Table 4-12. Get SCSI Disk Information Command

| Member | Description |  |
| --- | --- | --- |
| SRB_Cmd | [IN] This field must contain SC_GET_DISK_INFO. |  |
| SRB_Status | [OUT] On return, this field will be the same as the return status defined below. |  |
| SRB_HaId | [IN] This field specifies which installed host adapter the request is intended for. Host adapter numbers are always assigned by the ASPI manager layer beginning with zero. |  |
| SRB_Flags | [IN] Reserved = 0 |  |
| SRB_Hdr_Rsvd | Reserved = 0 |  |
| SRB_Target | [IN] SCSI ID of target device. |  |
| SRB_Lun | [IN] Logical Unit Number (LUN) of device. This field is ignored by ASPI for Win32, since SCSI Bus Device resets are done on a per-target basis only. |  |
| SRB_DriveFlags | [OUT] Upon completion of the SCSI command, the ASPI manager sets this field as follows: |  |
|  | Bit 7-2 | Reserved |
|  | Bit 1-0 | These bits are encoded to reflect Int 13h control and accessibility to the device. Bit 1 is the most significant bit. DISK_NOT_INT13 Device is not controlled by Int 13h services DISK_INT13_AND_DOS Device is under Int 13h control and is claimed by DOS DISK_INT1 Device is under Int 13h control but not claimed by DOS |
| SRB_Int13DriveInfo | [OUT] Upon completion of the SCSI command, the ASPI manager sets this field with the physical drive number that Int 13h services assigned to the device. The valid drive numbers are 0x00 to 0xFF. This field is only valid if bits 1-0 of the SRB_DriveFlags were set to DISK_INT13_AND_DOS or DISK_INT13. |  |
| SRB_Heads | [OUT] Upon completion of the SCSI command, the ASPI manager sets this field to the number of heads the Int13h services is using for this device’s geometry. The valid drive numbers are 0x00 to 0xFF. This field is only valid if bits 1-0 of the SRB_DriveFlags were set to DISK_INT13_AND_DOS or DISK_INT13. |  |
| SRB_Sectors | [OUT] Upon completion of the SCSI command, the ASPI manager sets this field to the number of sectors the Int 13h services is using for this device’s geometry. The valid drive numbers are 0x00 to 0xFF. This field is only valid if bits 1-0 of the SRB_DriveFlags were set to DISK_INT13_AND_DOS or DISK_INT13. |  |

Table 4-13. Return Values from Get SCSI Disk Information Command

| Value | Meaning |
| --- | --- |
| SS_COMP | SCSI/ASPI request has completed without error. |
| SS_INVALID_HA | Invalid host adapter number. |
| SS_INVALID_SRB | One or more parameters in the SCSI Request Block (SRB) are set incorrectly. |

Example

This example obtains disk information from device LUN 0, SCSI ID 2, attached to host adapter #0.

SRB_GetDiskInfo MySRB;

DWORD ASPIStatus;

.

.

MySRB.SRB_Header.SRB_Cmd	= SC_GET_DISK_INFO;

MySRB.SRB_HaId				= 0;

MySRB.SRB_Flags				= 0;

MySRB.SRB_Hdr_Rsvd			= 0;

MySRB.SRB_Target				= 2;

MySRB.SRB_Lun				= 0;

ASPIStatus					= SendASPI32Command ( (LPSRB) &MySRB );

.

.

Request Completion/Notification

Once an ASPI for Win32 SCSI request has been sent to the ASPI manager, there are three ways of being notified that the SCSI request has completed. The first and recommended method uses event notification. The second method uses callback (also referred to as posting). The third method uses polling.

Using Event Notification

Event notification is an ideal mechanism for notifying ASPI clients of the completion of an ASPI request. ASPI clients may block on this event until completion. Upon completion of a request, the ASPI for Win32 manager sets the event to the signaled state. The ASPI client must make sure that the event is not in a signaled state when an ASPI command request is submitted to the ASPI for Win32 manager. Event notification is illustrated in the following code, which sends a SCSI Inquiry command to target #2.

SRB_ExecSCSICmd MySRB;

HANDLE ASPICompletionEvent;

DWORD ASPIEventStatus;

DWORD ASPIStatus;

char InquiryBuffer[32];

.

/**************************************************/

/* Create event for MySRB.  Initial state         */

/* non-signaled, manual reset.                    */

/**************************************************/

if ((ASPICompletionEvent = CreateEvent(NULL,FALSE,FALSE,NULL)) == NULL)

return FALSE;

/**************************************************/

/* Code is entered with 'MySRB' zeroed.           */

/**************************************************/

MySRB.SRB_Cmd					= SC_EXEC_SCSI_CMD;

MySRB.SRB_Flags					= SRB_DIR_SCSI|SRB_EVENT_NOTIFY;

MySRB.SRB_Target					= 2;

MySRB.SRB_BufLen					= 32;

MySRB.SRB_SenseLen				= SENSE_LEN;

MySRB.SRB_BufPointer			= InquiryBuffer;

MySRB.SRB_CDBLen					= 6;

MySRB.CDBByte[0]					= SCSI_INQUIRY;

MySRB.CDBByte[4]					= 32;

MySRB.SRB_PostProc				= ASPICompletionEvent;

.

.

ASPIStatus						= SendASPI32Command ( (LPSRB) &MySRB );

/**************************************************/

/* Block on event till signaled                   */

/**************************************************/

if ( MySRB.SRB_Status == SS_PENDING )

ASPIEventStatus == WaitForSingleObject(ASPICompletionEvent, TIMEOUT);

/**************************************************/

/* Reset event to non-signaled state.             */

/**************************************************/

if (ASPIEventStatus == WAIT_OBJECT_0)

ResetEvent(ASPICompletionEvent);

.

.

Using Callback

Callback (or posting) may also be used to receive notification that a SCSI request has completed. When callback is used, ASPI for Win32 posts completion by passing control to your callback function. For example, the following code segment sends a SCSI Inquiry command to target #2 during the WM_CREATE message.

LRESULT CALLBACK WndProc (HWND, UINT, UPARAM, LPARAM);

void APIENTRY ASPIPostProc (PSRB_ExecSCSICmd );

HWND		PostHWND;

HANDLE		hInstance;

.

.

.

//****************************************************************************

//

// Function:		ASPIPostProc

//

// Description:	If POSTING is enabled,  this function is called by ASPI

//					for Windows when the SCSI request has completed.  This

//					sample function simply posts a message to our Window

//					handle to indicate that the SCSI request has completed.

//

//	DoneSRB		This parameter points to the ASPI SCSI Request Block (SRB)

//					which has completed.

//

//	Returns  		Nothing

//

//****************************************************************************

#ifdef WIN32

void ASPIPostProc(PSRB_ExecSCSICmd DoneSRB)

{

PostMessage(PostHWND,WM_ASPIPOST,0,(LPARAM)DoneSRB);

return;

}

#else

void _loadds __far __pascal ASPIPostProc(LPSRB DoneSRB)

{

PostMessage(PostHWND,WM_ASPIPOST,

(WORD)((SRB_ExecSCSICmd6 far *)DoneSRB)->SRB_Status,

(DWORD)DoneSRB);

return;

}

#endif

// **************************************************************************

// *

// *	Procedure:		WndProc()

// *

// **************************************************************************

#ifdef WIN32

LRESULT CALLBACK WndProc(HWND hwnd,UINT message,WPARAM wParam,
	LPARAM lParam)

#else

long __far __pascal _export WndProc(HWND hwnd,UINT message,WPARAM
	wParam,LPARAM lParam)

#endif

{

HDC         		hdc;

PAINTSTRUCT 		ps;

TEXTMETRIC		tm;

HMENU				hMenu;

static short		cxClient, cyClient;

int 				i=0;

WORD				status;

#ifdef WIN32

SRB_ExecSCSICmd *SRBPtr;

#else

SRB_ExecSCSICmd6 far *SRBPtr;

#endif

switch (message)

{

case WM_CREATE:

hdc = GetDC(hwnd);

SelectObject(hdc,GetStockObject(SYSTEM_FIXED_FONT));

GetTextMetrics(hdc,&tm);

cxChar = tm.tmAveCharWidth;

cyChar = tm.tmHeight;

ReleaseDC(hwnd,hdc);

rect.top = 0;

return 0;

case WM_PAINT:

InvalidateRect(hwnd,NULL,TRUE);

hdc = BeginPaint(hwnd,&ps) ;

EndPaint (hwnd,&ps) ;

return 0 ;

case WM_SIZE:

cxClient = LOWORD(lParam);

cyClient = HIWORD(lParam);

rect.right 	= LOWORD(lParam);

rect.bottom = HIWORD(lParam);

UpdateWindow(hwnd);

return 0;

case WM_COMMAND:

hMenu = GetMenu(hwnd);

switch (wParam)

{

case ID_FILE_EXIT:

SendMessage(hwnd,WM_CLOSE,0,0L);

return 0;

default:

// 10 = target #0

// 11 = target #1

// 12 = target #2

// 13 = target #3

// 14 = target #4

// 15 = target #5

// 16 = target #6

// 17 = target #7

// Toggle the SCSI target scan status (ENABLED/DISABELD)

if ((wParam >= 10 && wParam <= 17) ||

(wParam >= 20 && wParam <= 27))

{

status = GetMenuState(hMenu,wParam,MF_BYCOMMAND) & MF_CHECKED;

CheckMenuItem(hMenu,wParam,MF_BYCOMMAND | ((status) ? MF_UNCHECKED:MF_CHECKED));

}

return 0;

}

case WM_ASPIPOST:

#ifdef WIN32

SRBPtr = (PSRB_ExecSCSICmd)lParam;

#else

SRBPtr = (SRB_ExecSCSICmd6 far *)lParam;

#endif

DisplaySCSIID(hwnd,SRBPtr);

ScanNextTarget(hwnd);

return 0;

case WM_DESTROY:

PostQuitMessage(0) ;

return 0 ;

}

return DefWindowProc (hwnd, message, wParam, lParam) ;

}

When the post routine is called, this sample post handler will fill the wParam field with the status of ASPI command (SRB_Status) while the lParam field will contain a pointer to the SRB which has completed.

Polling

Polling is another method of determining SCSI request completion. This method is not recommended. After the command is sent and ASPI for Win32 returns control back to the calling application, you can then poll the status byte waiting for the command to complete. For example, the following code segment will execute a SCSI Inquiry command to target #2 using polling.

SRB_ExecSCSICmd6 MySRB;

DWORD ASPIStatus;

char InquiryBuffer[32];

.

.

/**************************************************/

/* Code is entered with 'MySRB' zeroed.

/**************************************************/

MySRB.SRB_Cmd					= SC_EXEC_SCSI_CMD;

MySRB.SRB_Flags					= SRB_DIR_SCSI;

MySRB.SRB_Target					= 2;

MySRB.SRB_BufLen					= 32;

MySRB.SRB_SenseLen				= SENSE_LEN;

MySRB.SRB_BufPointer			= InquiryBuffer;

MySRB.SRB_CDBLen					= 6;

MySRB.CDBByte[0]					= SCSI_INQUIRY;

MySRB.CDBByte[4]					= 32;

.

.

ASPIStatus						= SendASPI32Command ( (LPSRB) &MySRB );

// Send Inquiry command

while ( MySRB.SRB_Status == SS_PENDING );			// Wait till it's finished

Migrating to ASPI for Win32

If you have written ASPI or ASPI for Windows applications in the past, here are some key points to keep in mind when you migrate the applications to ASPI for Win32:

- Although structure definitions have changed slightly to optimize alignment for 32-bit processors, structure names are consistent with those used in previous ASPI developer’s kits. If you would like to use one source base for both 16-bit and 32-bit applications, make sure that you conditionally compile with the appropriate include files for each programming model. New 32-bit include files are available in the ASPI developer’s kit. There are many #ifdef statements in the sample code.

- The CDB area has been fixed in length at 16. Therefore, the sense data area no longer shifts location depending on command length. If you are developing an application targeted only at Win32, you no longer need to account for the “floating” sense buffer.

- For requests requiring data transfers, the direction bits in the SRB Flags field must be set correctly. Direction bits are no longer optional for data transfers. For requests not requiring data transfers, the direction bits are ignored.

- If you are using dynamic DLL linking, use LoadLibrary (winaspi32.dll) instead of LoadLibrary(winaspi.dll) in order to access ASPI for Win32. If you are using static DLL linking, use the wnaspi32.lib.

- All pointers in Win32 are 32 bits (Flat model). Therefore, FAR references are not used.

- When allocating memory, page locking and global allocations are not necessary.

Programming Guidelines

The following are additional guidelines for writing ASPI for Win32 applications.

- ASPI for Win32 applications/clients should send an ASPI Abort command to terminate pending ASPI requests before exit.

- SRBs and data buffers do not need to be in page-locked memory. The ASPI manager takes care of locking buffers and SRBs.

- If an error SS_BUFFER_TO_BIG is returned by the SendASPI32Command routine, you should break the transfer down into 64 KByte transfers or less. For maximum compatibility, we recommend that you do not request transfer sizes larger than 64 KBytes unless your application requires it.

- If you send an ASPI request with posting or callback enabled, the callback procedure will always be called. The post or callback routine is called as a standard C function. The caller (in this case, the ASPI manager) cleans up the stack.

- ASPI for Win32 is fully re-entrant and permits overlapped, asynchronous I/O. Clients can send additional ASPI requests while others are pending completion. Be sure to use a separate SRB for each ASPI request.

- Be sure to zero out all reserved fields before passing an SRB to ASPI for Win32.

- Be sure that buffers are aligned according to the buffer alignment mask returned by the SC_HA_INQUIRY command. An alignment of at least a double word is recommended.

- All SRB structures sent to SendASPI32Command should be compiled packed with byte alignment.

- Some SCSI devices may have security restrictions that make them inaccessible for read/write operations. In operating systems where security is a feature, accessing a device where the application does not have sufficient security privilege results in a SS_SECURITY_VIOLATION error.

- When scanning for devices, the SendASPI32Command may also return the status SS_NO_DEVICE in the SRB_Status field. Check for this exception in addition to the host adapter status HASTAT_SEL_TO. As previously mentioned, the callback procedure is always called, including this condition.

- The SRB flags SRB_POSTING and SRB_EVENT_NOTIFY are mutually exclusive.

Scanning the SCSI Bus for Devices

scanscsi.exe is a very simple ASPI for Win16 and ASPI for Win32 application which scans the SCSI bus and displays the device name of each target it finds on the SCSI bus. When the program first comes up, you are prompted to select either ASPI posting or polling to detect completion of a SCSI request. The Scan menu enables you to configure which SCSI targets should be scanned and which should not:

The following scanscsi.c sample code also shows you how to free up the processor while SCSI operations are running in the background. This is very important, since Windows is currently a non-preemptive multitasking operating system. scanscsi.exe does minimal error checking and has no time-out code. Your application should be more robust.

Source Code for scanscsi.c

// **************************************************************************

//

//									Adaptec, Inc.

//

//								 ASPI Developer's  Kit

//								  Copyright 1992-1994

//

// **************************************************************************

//

//		This software contains the valuable trade secrets of Adaptec.

//		The software is protected under copyright laws as an

//		unpublished work of Adaptec.  Notice is for informational

//		purposes only and does not imply publication.  The user of this

//		software may make copies of the software for use with parts

//		manufactures by Adaptec or under license from Adaptec and for

//		no other use.

//

// **************************************************************************

//

//		Program Names:		SCAN16.EXE				SCAN32.EXE

//

//		Version:				v2.0						v2.0

//

//		Source Code:			SCANSCSI.C				SCANSCSI.C

//								SCANSCSI.RC				SCANSCSI.RC

//								RESOURCE.H				RESOURCE.H

//								SCAN16.DEF				SCAN32.DEF

//								SCAN16.MAK				SCAN32.MAK

//								SCSIDEFS.H				SCSIDEFS.H

//								WINASPI.H					WNASPI32.H

//

//		Description:			The ASPI test utility programs included here

//								will run on Windows 3.0 or later.  The utility

//								SCAN32.EXE was written for Windows 95 using the

//								ASPI for WIN32 specifications.  Both programs

//								open a window and loop continuously while

//								displaying all of the SCSI devices found on the

//								SCSI bus.  The user can exit at any time by

//								pressing ALT F4 or by selecting EXIT from the

//								applications menu.  The user can enable and

//								disable, on a per target basis, which SCSI targets

//								should get scanned.

//								You should have a basic understanding of writing

//								Windows applications before looking at this

//								source code.

//								Note that this sample code does minimal error

//								checking and no timeout handling.  Your application

//								should be more robust.  This code also demonstrates

//								the polling and posting techniques of determining

//								the completion of the SCSI command.  REFER TO THE

//								"ASPI FOR WINDOWS" SPECIFICATION FOR MORE

//								INFORMATION.

//

//

// **************************************************************************

#pragma pack(1)

#include <windows.h>

#include <string.h>

#include "resource.h"

#include "scsidefs.h"

#ifdef WIN32

#include "wnaspi32.h"

#else

#include "winaspi.h"

#endif

#ifdef WIN32

LRESULT CALLBACK WndProc(HWND,UINT,WPARAM,LPARAM);

#else

long FAR PASCAL WndProc(HWND,UINT,UINT,LONG);

#endif

void ScanNextTarget(HWND);

#ifdef WIN32

void ASPIPostProc(PSRB_ExecSCSICmd);

void DisplaySCSIID (HWND,void *);

#else

void _loadds __far __pascal ASPIPostProc(LPSRB);

void DisplaySCSIID (HWND,SRB_ExecSCSICmd6 far *);

#endif

HWND		PostHWND;

WORD 		NumAdapters;

RECT 		rect;

int  		UsePosting;

char 		szAppName[] = "SCANSCSI" ;

int	 		cxChar,cyChar;

#ifndef WIN32

FARPROC			lpfnASPIPostProc;

#endif

// **************************************************************************

//

//  										WINMAIN

//

// **************************************************************************

int PASCAL WinMain(HANDLE hInstance, HANDLE hPrevInstance,

LPSTR lpszCmdParam, int nCmdShow)

{

HWND        hwnd;

MSG         msg ;

WNDCLASS    wndclass ;

#ifdef WIN32

DWORD 	ASPIStatus;

#else

WORD	ASPIStatus;

#endif

if (!hPrevInstance)

{

wndclass.style				= CS_HREDRAW | CS_VREDRAW ;

wndclass.lpfnWndProc		= WndProc ;

wndclass.cbClsExtra 		= 0 ;

wndclass.cbWndExtra			= 0 ;

wndclass.hInstance			= hInstance ;

wndclass.hIcon		  = LoadIcon (hInstance,MAKEINTRESOURCE(SCANSCSI)) ;

wndclass.hCursor				= LoadCursor (NULL, IDC_ARROW) ;

wndclass.hbrBackground		= GetStockObject (WHITE_BRUSH);

wndclass.lpszMenuName		= MAKEINTRESOURCE(Menu1);

wndclass.lpszClassName		= szAppName ;

RegisterClass (&wndclass) ;

}

hwnd = CreateWindow (

szAppName, 					// window class name

"SCANSCSI",					// window caption

WS_OVERLAPPEDWINDOW,		// window style

CW_USEDEFAULT,	 			// initial x position

CW_USEDEFAULT,				// initial y position

CW_USEDEFAULT,				// initial x size

CW_USEDEFAULT,				// initial y size

NULL,							// parent window handle

NULL,							// window menu handle

hInstance,					// program instance handle

NULL) ;						// creation parameters

ShowWindow (hwnd, nCmdShow) ;

// **************************************************************************

// *

// *		Here we check to make sure that ASPI for Windows is resident.

// *		If ASPI for Windows or ASPI for WIN32 is not resident it is

// *		suggested that you provide more user feedback/help than what is

// *		shown in this sample code.

// *

// **************************************************************************

#ifdef WIN32

ASPIStatus = GetASPI32SupportInfo();

switch (HIBYTE(LOWORD(ASPIStatus)))

#else

ASPIStatus = GetASPISupportInfo();

switch (HIBYTE(ASPIStatus))

#endif

{

case SS_COMP:						// ASPI for Windows is resident.

#ifdef WIN32

NumAdapters = (LOWORD(LOBYTE(ASPIStatus)));

#else

NumAdapters = LOBYTE(ASPIStatus);

#endif

MessageBox(hwnd,"ASPI for Windows is resident!",
						 szAppName,MB_ICONINFORMATION);

break;

default:

MessageBox(hwnd,"ASPI for Windows not available!",
						 szAppName,MB_ICONSTOP);

return 0;

}

// **************************************************************************

// *

// *		This code demonstrates the posting abilities of ASPI for Windows

// * 		as well as the polling technique.  Ask the user which method they

// *		would like to use to indicate completion of the SCSI request.

// *

// **************************************************************************

UsePosting = MessageBox(hwnd,"Use ASPI for Windows Posting?",

szAppName,

MB_YESNO | MB_ICONQUESTION);

#ifndef WIN32							// NOTE: No need to lock buffers in WIN32.

LockData(0);

lpfnASPIPostProc = MakeProcInstance ((FARPROC)ASPIPostProc, hInstance);

#endif

PostHWND = hwnd;

if (UsePosting == IDYES)

{

ScanNextTarget(hwnd);

while (GetMessage (&msg, NULL, 0, 0))

{

TranslateMessage (&msg) ;

DispatchMessage (&msg) ;

}

}

else

{

while (TRUE)

{

if (PeekMessage (&msg,NULL,0,0,PM_REMOVE))

{

if (msg.message == WM_QUIT)

break;

TranslateMessage(&msg);

DispatchMessage(&msg);

}

else								// While the processor is free, check

ScanNextTarget(hwnd);		// the SCSI bus status.

}

}

ScanNextTarget(0);						// Finish any pending ASPI requests

return msg.wParam ;

}

// **************************************************************************

// *

// *		Procedure:	WndProc()

// *

// **************************************************************************

#ifdef WIN32

LRESULT CALLBACK WndProc(HWND hwnd,UINT message,WPARAM wParam,
	LPARAM lParam)

#else

long __far __pascal _export WndProc(HWND hwnd,UINT message,WPARAM
	wParam,LPARAM lParam)

#endif

{

HDC         		hdc;

PAINTSTRUCT 		ps;

TEXTMETRIC		tm;

HMENU				hMenu;

static short		cxClient, cyClient;

int 				i=0;

WORD				status;

#ifdef WIN32

SRB_ExecSCSICmd *SRBPtr;

#else

SRB_ExecSCSICmd6 far *SRBPtr;

#endif

switch (message)

{

case WM_CREATE:

hdc = GetDC(hwnd);

SelectObject(hdc,GetStockObject(SYSTEM_FIXED_FONT));

GetTextMetrics(hdc,&tm);

cxChar = tm.tmAveCharWidth;

cyChar = tm.tmHeight;

ReleaseDC(hwnd,hdc);

rect.top = 0;

return 0;

case WM_PAINT:

InvalidateRect(hwnd,NULL,TRUE);

hdc = BeginPaint(hwnd,&ps) ;

EndPaint (hwnd,&ps) ;

return 0 ;

case WM_SIZE:

cxClient = LOWORD(lParam);

cyClient = HIWORD(lParam);

rect.right 	= LOWORD(lParam);

rect.bottom = HIWORD(lParam);

UpdateWindow(hwnd);

return 0;

case WM_COMMAND:

hMenu = GetMenu(hwnd);

switch (wParam)

{

case ID_FILE_EXIT:

SendMessage(hwnd,WM_CLOSE,0,0L);

return 0;

default:

// 10 = target #0

// 11 = target #1

// 12 = target #2

// 13 = target #3

// 14 = target #4

// 15 = target #5

// 16 = target #6

// 17 = target #7

// Toggle the SCSI target scan status (ENABLED/DISABELD)

if ((wParam >= 10 && wParam <= 17) ||

(wParam >= 20 && wParam <= 27))

{

status = GetMenuState(hMenu,wParam,MF_BYCOMMAND) & 
								   MF_CHECKED;

CheckMenuItem(hMenu,wParam,MF_BYCOMMAND | ((status) ? 
						MF_UNCHECKED:MF_CHECKED));

}

return 0;

}

case WM_ASPIPOST:

#ifdef WIN32

SRBPtr = (PSRB_ExecSCSICmd)lParam;

#else

SRBPtr = (SRB_ExecSCSICmd6 far *)lParam;

#endif

DisplaySCSIID(hwnd,SRBPtr);

ScanNextTarget(hwnd);

return 0;

case WM_DESTROY:

PostQuitMessage(0) ;

return 0 ;

}

return DefWindowProc (hwnd, message, wParam, lParam) ;

}

// **************************************************************************

//

//		Function:		ScanNextTarget

//

// Description:		If polling is enabled, this function is called

//						whenever Windows is idle.  If a SCSI Inquiry command

//						is still in progress,  this routine will return

//						control back to the caller to free up the CPU. If

//						POSTING is enabled,  this function is called when the

//						SCSI Inquiry command has completed.

//

//		hwnd -			This parameter contains the Window handle where the

//						device names should be listed.  If hwnd equals zero,

//						this indicates that we're exiting this program and this

//						routine should return only after all outstanding ASPI

//						requests have completed.

//

// Returns -			Nothing

//

// **************************************************************************

void ScanNextTarget(HWND hwnd)

{

HMENU				hMenu;

static WORD		SRBIsPending=FALSE;

static BYTE		target_id=7,adapter_id=0;

static char		InquiryBuffer[100];

WORD				status, j;

#ifdef WIN32

static		SRB_ExecSCSICmd ExecSRB;

#else

static		SRB_ExecSCSICmd6 ExecSRB;

#endif

//	If the user is exiting the program, wait

//	until all pending ASPI requests are

if (hwnd==0)					// 	finished.

{

if (SRBIsPending==TRUE)

{

while (ExecSRB.SRB_Status == SS_PENDING);

}

return;

}

// ***************************************************************************

// -

// - 	If we are still waiting for the previous SCSI Inquiry to finish,

// -	simply return back to the caller to free up the CPU.

// -

// ***************************************************************************

if (UsePosting==IDNO && SRBIsPending==TRUE && 
		ExecSRB.SRB_Status==SS_PENDING)

{

return;

}

if (SRBIsPending==FALSE || UsePosting==IDYES)

{

hMenu = GetMenu(hwnd);

target_id = (target_id + 1)%8;	// Move to next SCSI ID#

if (target_id == 0)

{

adapter_id++;

if (adapter_id >= (BYTE)NumAdapters)

adapter_id = 0;

}

// Here is where we check

for (j=0;j<8;j++)					// the menu status.

{

status = GetMenuState(hMenu,target_id+10,MF_BYCOMMAND) &
					   MF_CHECKED;

if (status == 0)

{

target_id = (target_id + 1) % 8;		// Move to next SCSI ID#

if (target_id == 0)

{

adapter_id++;

if (adapter_id = (BYTE)NumAdapters)

{

adapter_id = 0;

}

}

}

else

break;

}

// Now we construct the SCSI Inquiry SRB and send it to ASPI!

if (j == 8)						//	Return if user disabled scanning of all

{									// 	SCSI targets.

return;

}

ExecSRB.SRB_Cmd					= SC_EXEC_SCSI_CMD;

ExecSRB.SRB_HaId					= adapter_id;

ExecSRB.SRB_Flags				= SRB_DIR_IN | SRB_DIR_SCSI;

ExecSRB.SRB_Target				= target_id;

ExecSRB.SRB_BufPointer			= InquiryBuffer;

ExecSRB.SRB_BufLen				= 32;

ExecSRB.SRB_SenseLen			= SENSE_LEN;

ExecSRB.SRB_CDBLen				= 6;

ExecSRB.CDBByte[0]				= SCSI_INQUIRY;

ExecSRB.CDBByte[1]				= 0;

ExecSRB.CDBByte[2]				= 0;

ExecSRB.CDBByte[3]				= 0;

ExecSRB.CDBByte[4]				= 32;

ExecSRB.CDBByte[5]				= 0;

if (UsePosting == IDYES)

{

ExecSRB.SRB_Flags  |= SRB_POSTING;

#ifdef WIN32

ExecSRB.SRB_PostProc = (void *)ASPIPostProc;

#else

ExecSRB.SRB_PostProc = lpfnASPIPostProc;

#endif

}

#ifdef WIN32

SendASPI32Command(&ExecSRB);

#else

SendASPICommand((LPSRB)&ExecSRB);

#endif

SRBIsPending = TRUE;					// ASPI command in process

return;

}

if (UsePosting == IDNO)

{

SRBIsPending=FALSE;						// Inquiry command has completed

DisplaySCSIID(hwnd,&ExecSRB);

}

return;

}

// **************************************************************************

// Function:		DisplaySCSIID

//

// Description:	This function will scroll the window down and display

//					the device name of the target we just sent a SCSI Inquiry

//					command.

//

// hwnd    	-	This parameter contains the window handle where the device

//					name should be displayed.

//

// ExecSRB 	- 	This parameter points to the ASPI SCSI Request Block (SRB)

//					which has completed.

//

// Returns:		Nothing

// **************************************************************************

#ifdef WIN32

void DisplaySCSIID ( HWND hwnd, SRB_ExecSCSICmd *ExecSRB)

#else

void DisplaySCSIID(HWND hwnd,SRB_ExecSCSICmd6 far *ExecSRB)

#endif

{

HDC			hdc;

char		string_buf[80];

int			string_len;

hdc = GetDC (hwnd);

ScrollDC (hdc, 0, -cyChar, &rect, &rect, NULL, NULL);

SelectObject (hdc, GetStockObject (SYSTEM_FIXED_FONT));

if (ExecSRB->SRB_Status==SS_COMP)

{

wsprintf (	string_buf,

"Host Adapter #%d - SCSI ID #%d:  ",

(BYTE) ExecSRB->SRB_HaId,

(BYTE) ExecSRB->SRB_Target );

#ifdef WIN32

strncat(string_buf,&ExecSRB->SRB_BufPointer[8],28);

#else

lstrcat(string_buf,&ExecSRB->SRB_BufPointer[8] );

#endif

string_len = lstrlen(string_buf);

}

else

{

string_len = wsprintf (string_buf,

"Host Adapter #%d - SCSI ID #%d:  %-32s",

(BYTE)ExecSRB->SRB_HaId,

(BYTE)ExecSRB->SRB_Target,

(LPSTR)"No Device");

}

TextOut(hdc,cxChar,rect.bottom-cyChar,string_buf,string_len);

ReleaseDC(hwnd,hdc);

ValidateRect(hwnd,NULL);

return;

}

// ***************************************************************************

//

// Function:			ASPIPostProc

//

// Description:		If POSTING is enabled,  this function is called by ASPI

//						for Windows when the SCSI request has completed.  This

//						sample function simply posts a message to our Window

//						handle to indicate that the SCSI request has completed.

//

//	DoneSRB			This parameter points to the ASPI SCSI Request Block (SRB)

//						which has completed.

//

//	Returns  			Nothing

//****************************************************************************

#ifdef WIN32

void ASPIPostProc(PSRB_ExecSCSICmd DoneSRB)

{

PostMessage(PostHWND,WM_ASPIPOST,0,(LPARAM)DoneSRB);

return;

}

#else

void _loadds __far __pascal ASPIPostProc(LPSRB DoneSRB)

{

PostMessage	(PostHWND,WM_ASPIPOST,

(WORD)((SRB_ExecSCSICmd6 far *)DoneSRB)->SRB_Status,

(DWORD)DoneSRB);

return;

}

#endif

Source Code for scan32.def

This module definition file is used for scanscsi in the Win32 environment.

NAME 			  SCAN32

DESCRIPTION    'SCANSCSI Program'

CODE           PRELOAD MOVEABLE DISCARDABLE

DATA           PRELOAD MOVEABLE MULTIPLE

HEAPSIZE       1024

ASPI for Win32 Definitions (wnaspi32.h)

//***************************************************************************

//

// Name: 			WNASPI32.H

//

// Description:	ASPI for Win32 definitions ('C' Language)

//

//***************************************************************************

#ifdef __cplusplus

extern "C" {

#endif

typedef void *LPSRB;

typedef void (*PFNPOST)();

DWORD SendASPI32Command    (LPSRB);

DWORD GetASPI32SupportInfo (VOID);

#define SENSE_LEN						14			// Default sense buffer length

#define SRB_DIR_IN						0x08		// Transfer from SCSI target to

// host

#define SRB_DIR_OUT					0x10		// Transfer from host to SCSI

// target

#define SRB_POSTING					0x01		// Enable ASPI posting

#define SRB_EVENT_NOTIFY				0x40		// Enable ASPI event notification

#define SRB_ENABLE_RESIDUAL_COUNT		0x04		// Enable residual byte count

// reporting

#define SRB_DATA_SG_LIST				0x02		// Data buffer points to

// scatter-gather list

#define WM_ASPIPOST					0x4D42		// ASPI Post message

//***************************************************************************

//						 %%% ASPI Command Definitions %%%

//***************************************************************************

#define SC_HA_INQUIRY				0x00			// Host adapter inquiry

#define SC_GET_DEV_TYPE			0x01			// Get device type

#define SC_EXEC_SCSI_CMD			0x02			// Execute SCSI command

#define SC_ABORT_SRB				0x03			// Abort an SRB

#define SC_RESET_DEV				0x04			// SCSI bus device reset

#define SC_GET_DISK_INFO			0x06			// Get Disk information

//***************************************************************************

//								  %%% SRB Status %%%

//***************************************************************************

#define SS_PENDING					0x00			// SRB being processed

#define SS_COMP					0x01			// SRB completed without error

#define SS_ABORTED					0x02			// SRB aborted

#define SS_ABORT_FAIL				0x03			// Unable to abort SRB

#define SS_ERR 					0x04			// SRB completed with error

#define SS_INVALID_CMD				0x80			// Invalid ASPI command

#define SS_INVALID_HA				0x81			// Invalid host adapter number

#define SS_NO_DEVICE				0x82			// SCSI device not installed

#define SS_INVALID_SRB				0xE0			// Invalid parameter set in SRB

#define SS_BUFFER_ALIGN			0xE1			// Buffer alignment problem

#define SS_SECURITY_VIOLATION		0xE2			// Device access security violation

#define SS_FAILED_INIT				0xE4			// ASPI for windows failed init

#define SS_BUFFER_TO_BIG			0xE6			// Buffer size too big to handle!

//***************************************************************************

//							%%% Host Adapter Status %%%

//***************************************************************************

#define HASTAT_OK					0x00			// Host adapter did not detect an

// error

#define HASTAT_SEL_TO				0x11			// Selection Timeout

#define HASTAT_DO_DU				0x12			// Data overrun data underrun

#define HASTAT_BUS_FREE			0x13			// Unexpected bus free

#define HASTAT_PHASE_ERR			0x14			// Target bus phase sequence

// failure

#define HASTAT_TIMEOUT				0x09			// Timed out while SRB was

// waiting to beprocessed.

#define HASTAT_COMMAND_TIMEOUT 	0x0B			// While processing the SRB, the

// adapter timed out.

#define HASTAT_MESSAGE_REJECT		0x0D			// While processing SRB, the

// adapter received a MESSAGE

// REJECT.

#define HASTAT_BUS_RESET			0x0E			// A bus reset was detected.

#define HASTAT_PARITY_ERROR		0x0F			// A parity error was detected.

#define HASTAT_REQUEST_SENSE_FAILED 0x10			// The adapter failed in issuing

// REQUEST SENSE.

//***************************************************************************

//			 %%% SRB - HOST ADAPTER INQUIRY - SC_HA_INQUIRY %%%

//***************************************************************************

typedef struct {

BYTE		SRB_Cmd;				// ASPI command code = SC_HA_INQUIRY

BYTE		SRB_Status;				// ASPI command status byte

BYTE		SRB_HaId;				// ASPI host adapter number

BYTE		SRB_Flags;				// ASPI request flags

DWORD		SRB_Hdr_Rsvd;			// Reserved, MUST = 0

BYTE		HA_Count;				// Number of host adapters present

BYTE		HA_SCSI_ID;				// SCSI ID of host adapter

BYTE		HA_ManagerId[16];		// String describing the manager

BYTE		HA_Identifier[16];		// String describing the host adapter

BYTE		HA_Unique[16];			// Host Adapter Unique parameters

WORD		HA_Rsvd1;

} SRB_HAInquiry, *PSRB_HAInquiry;

//***************************************************************************

//			  %%% SRB - GET DEVICE TYPE - SC_GET_DEV_TYPE %%%

//***************************************************************************

typedef struct {

BYTE		SRB_Cmd;				// ASPI command code = SC_GET_DEV_TYPE

BYTE		SRB_Status;				// ASPI command status byte

BYTE		SRB_HaId;				// ASPI host adapter number

BYTE		SRB_Flags;				// Reserved

DWORD		SRB_Hdr_Rsvd;			// Reserved

BYTE		SRB_Target;				// Target's SCSI ID

BYTE		SRB_Lun;				// Target's LUN number

BYTE		SRB_DeviceType;		// Target's peripheral device type

BYTE		SRB_Rsvd1;

} SRB_GDEVBlock, *PSRB_GDEVBlock;

//***************************************************************************

//		  %%% SRB - EXECUTE SCSI COMMAND - SC_EXEC_SCSI_CMD %%%

//***************************************************************************

typedef struct {

BYTE		SRB_Cmd;				// ASPI command code = SC_EXEC_SCSI_CMD

BYTE		SRB_Status;				// ASPI command status byte

BYTE		SRB_HaId;				// ASPI host adapter number

BYTE		SRB_Flags;				// ASPI request flags

DWORD		SRB_Hdr_Rsvd;			// Reserved

BYTE		SRB_Target;				// Target's SCSI ID

BYTE		SRB_Lun;				// Target's LUN number

WORD		SRB_Rsvd1;				// Reserved for Alignment

DWORD		SRB_BufLen;				// Data Allocation Length

BYTE		*SRB_BufPointer;		// Data Buffer Pointer

BYTE		SRB_SenseLen;			// Sense Allocation Length

BYTE		SRB_CDBLen;				// CDB Length

BYTE		SRB_HaStat;				// Host Adapter Status

BYTE		SRB_TargStat;			// Target Status

void		(*SRB_PostProc)();		// Post routine

void		*SRB_Rsvd2;				// Reserved

BYTE		SRB_Rsvd3[16];			// Reserved for alignment

BYTE		CDBByte[16];			// SCSI CDB

BYTE		SenseArea[SENSE_LEN+2];	// Request Sense buffer

} SRB_ExecSCSICmd, *PSRB_ExecSCSICmd;

//***************************************************************************

//				  %%% SRB - ABORT AN SRB - SC_ABORT_SRB %%%

//***************************************************************************

typedef struct {

BYTE		SRB_Cmd;				// ASPI command code = SC_EXEC_SCSI_CMD

BYTE		SRB_Status;				// ASPI command status byte

BYTE		SRB_HaId;				// ASPI host adapter number

BYTE		SRB_Flags;				// Reserved

DWORD		SRB_Hdr_Rsvd;			// Reserved

void		*SRB_ToAbort;			// Pointer to SRB to abort

} SRB_Abort, *PSRB_Abort;

//***************************************************************************

//				%%% SRB - BUS DEVICE RESET - SC_RESET_DEV %%%

//***************************************************************************

typedef struct {

BYTE		SRB_Cmd;				// ASPI command code = SC_EXEC_SCSI_CMD

BYTE		SRB_Status;				// ASPI command status byte

BYTE		SRB_HaId;				// ASPI host adapter number

BYTE		SRB_Flags;				// Reserved

DWORD		SRB_Hdr_Rsvd;			// Reserved

BYTE		SRB_Target;				// Target's SCSI ID

BYTE		SRB_Lun;				// Target's LUN number

BYTE		SRB_Rsvd1[12];			// Reserved for Alignment

BYTE		SRB_HaStat;				// Host Adapter Status

BYTE		SRB_TargStat;			// Target Status

void		(*SRB_PostProc)();		// Post routine

void		*SRB_Rsvd2;				// Reserved

BYTE		SRB_Rsvd3[32];			// Reserved

} SRB_BusDeviceReset, *PSRB_BusDeviceReset;

//***************************************************************************

//				%%% SRB - GET DISK INFORMATION - SC_GET_DISK_INFO %%%

//***************************************************************************

typedef struct {

BYTE		SRB_Cmd;				// ASPI command code = SC_EXEC_SCSI_CMD

BYTE		SRB_Status;				// ASPI command status byte

BYTE		SRB_HaId;				// ASPI host adapter number

BYTE		SRB_Flags;				// Reserved

DWORD		SRB_Hdr_Rsvd;			// Reserved

BYTE		SRB_Target;				// Target's SCSI ID

BYTE		SRB_Lun;				// Target's LUN number

BYTE		SRB_DriveFlags;		// Driver flags

BYTE		SRB_Int13HDriveInfo	// Host Adapter Status

BYTE		SRB_Heads;				// Preferred number of heads translation

BYTE		SRB_Sectors;			// Preferred number of sectors translation

BYTE		SRB_Rsvd1[10];			// Reserved

} SRB_GetDiskInfo, *PSRB_GetDiskInfo;

#ifdef __cplusplus

}

#endif

SCSI Definitions (scsidefs.h)

//***************************************************************************

//

// Name: 			SCSIDEFS.H

//

// Description:	SCSI definitions ('C' Language)

//

//***************************************************************************

//***************************************************************************

//							%%% TARGET STATUS VALUES %%%

//***************************************************************************

#define STATUS_GOOD				0x00			// Status Good

#define STATUS_CHKCOND				0x02			// Check Condition

#define STATUS_CONDMET				0x04			// Condition Met

#define STATUS_BUSY				0x08			// Busy

#define STATUS_INTERM				0x10			// Intermediate

#define STATUS_INTCDMET			0x14			// Intermediate-condition met

#define STATUS_RESCONF				0x18			// Reservation conflict

#define STATUS_COMTERM				0x22			// Command Terminated

#define STATUS_QFULL				0x28			// Queue full

//***************************************************************************

//						%%% SCSI MISCELLANEOUS EQUATES %%%

//***************************************************************************

#define MAXLUN						7				// Maximum Logical Unit Id

#define MAXTARG					7				// Maximum Target Id

#define MAX_SCSI_LUNS				64				// Maximum Number of SCSI LUNs

#define MAX_NUM_HA					8				// Maximum Number of SCSI HA's

//---------------------------------------------------------------------------

//

//							%%% SCSI COMMAND OPCODES %%%

//

//---------------------------------------------------------------------------

//***************************************************************************

//				 %%% Commands for all Device Types %%%

//***************************************************************************

#define SCSI_CHANGE_DEF			0x40			// Change Definition (Optional)

#define SCSI_COMPARE				0x39			// Compare (O)

#define SCSI_COPY					0x18			// Copy (O)

#define SCSI_COP_VERIFY			0x3A			// Copy and Verify (O)

#define SCSI_INQUIRY				0x12			// Inquiry (MANDATORY)

#define SCSI_LOG_SELECT			0x4C			// Log Select (O)

#define SCSI_LOG_SENSE				0x4D			// Log Sense (O)

#define SCSI_MODE_SEL6				0x15			// Mode Select 6-byte (Device Specific)

#define SCSI_MODE_SEL10			0x55			// Mode Select 10-byte (Device Specific)

#define SCSI_MODE_SEN6				0x1A			// Mode Sense 6-byte (Device Specific)

#define SCSI_MODE_SEN10			0x5A			// Mode Sense 10-byte (Device Specific)

#define SCSI_READ_BUFF				0x3C			// Read Buffer (O)

#define SCSI_REQ_SENSE				0x03			// Request Sense (MANDATORY)

#define SCSI_SEND_DIAG				0x1D			// Send Diagnostic (O)

#define SCSI_TST_U_RDY				0x00			// Test Unit Ready (MANDATORY)

#define SCSI_WRITE_BUFF			0x3B			// Write Buffer (O)

//***************************************************************************

//			  %%% Commands Unique to Direct Access Devices %%%

//***************************************************************************

#define SCSI_COMPARE				0x39			// Compare (O)

#define SCSI_FORMAT				0x04			// Format Unit (MANDATORY)

#define SCSI_LCK_UN_CAC			0x36			// Lock Unlock Cache (O)

#define SCSI_PREFETCH				0x34			// Prefetch (O)

#define SCSI_MED_REMOVL			0x1E			// Prevent/Allow medium Removal (O)

#define SCSI_READ6					0x08			// Read 6-byte (MANDATORY)

#define SCSI_READ10				0x28			// Read 10-byte (MANDATORY)

#define SCSI_RD_CAPAC				0x25			// Read Capacity (MANDATORY)

#define SCSI_RD_DEFECT				0x37			// Read Defect Data (O)

#define SCSI_READ_LONG				0x3E			// Read Long (O)

#define SCSI_REASS_BLK				0x07			// Reassign Blocks (O)

#define SCSI_RCV_DIAG				0x1C			// Receive Diagnostic Results (O)

#define SCSI_RELEASE				0x17			// Release Unit (MANDATORY)

#define SCSI_REZERO				0x01			// Rezero Unit (O)

#define SCSI_SRCH_DAT_E			0x31			// Search Data Equal (O)

#define SCSI_SRCH_DAT_H			0x30			// Search Data High (O)

#define SCSI_SRCH_DAT_L			0x32			// Search Data Low (O)

#define SCSI_SEEK6					0x0B			// Seek 6-Byte (O)

#define SCSI_SEEK10				0x2B			// Seek 10-Byte (O)

#define SCSI_SEND_DIAG				0x1D			// Send Diagnostics (MANDATORY)

#define SCSI_SET_LIMIT				0x33			// Set Limits (O)

#define SCSI_START_STP				0x1B			// Start/Stop Unit (O)

#define SCSI_SYNC_CACHE			0x35			// Synchronize Cache (O)

#define SCSI_VERIFY				0x2F			// Verify (O)

#define SCSI_WRITE6				0x0A			// Write 6-Byte (MANDATORY)

#define SCSI_WRITE10				0x2A			// Write 10-Byte (MANDATORY)

#define SCSI_WRT_VERIFY			0x2E			// Write and Verify (O)

#define SCSI_WRITE_LONG			0x3F			// Write Long (O)

#define SCSI_WRITE_SAME			0x41			// Write Same (O)

//***************************************************************************

//			%%% Commands Unique to Sequential Access Devices %%%

//***************************************************************************

#define SCSI_ERASE					0x19			// Erase (MANDATORY)

#define SCSI_LOAD_UN				0x1b			// Load/Unload (O)

#define SCSI_LOCATE				0x2B			// Locate (O)

#define SCSI_RD_BLK_LIM			0x05			// Read Block Limits (MANDATORY)

#define SCSI_READ_POS				0x34			// Read Position (O)

#define SCSI_READ_REV				0x0F			// Read Reverse (O)

#define SCSI_REC_BF_DAT			0x14			// Recover Buffer Data (O)

#define SCSI_RESERVE				0x16			// Reserve Unit (MANDATORY)

#define SCSI_REWIND				0x01			// Rewind (MANDATORY)

#define SCSI_SPACE					0x11			// Space (MANDATORY)

#define SCSI_VERIFY_T				0x13			// Verify (Tape) (O)

#define SCSI_WRT_FILE				0x10			// Write Filemarks (MANDATORY)

//***************************************************************************

//				  %%% Commands Unique to Printer Devices %%%

//***************************************************************************

#define SCSI_PRINT					0x0A			// Print (MANDATORY)

#define SCSI_SLEW_PNT				0x0B			// Slew and Print (O)

#define SCSI_STOP_PNT				0x1B			// Stop Print (O)

#define SCSI_SYNC_BUFF				0x10			// Synchronize Buffer (O)

//***************************************************************************

//				 %%% Commands Unique to Processor Devices %%%

//***************************************************************************

#define SCSI_RECEIVE				0x08			// Receive (O)

#define SCSI_SEND					0x0A			// Send (O)

//***************************************************************************

//				%%% Commands Unique to Write-Once Devices %%%

//***************************************************************************

#define SCSI_MEDIUM_SCN			0x38			// Medium Scan (O)

#define SCSI_SRCHDATE10			0x31			// Search Data Equal 10-Byte (O)

#define SCSI_SRCHDATE12			0xB1			// Search Data Equal 12-Byte (O)

#define SCSI_SRCHDATH10			0x30			// Search Data High 10-Byte (O)

#define SCSI_SRCHDATH12			0xB0			// Search Data High 12-Byte (O)

#define SCSI_SRCHDATL10			0x32			// Search Data Low 10-Byte (O)

#define SCSI_SRCHDATL12			0xB2			// Search Data Low 12-Byte (O)

#define SCSI_SET_LIM_10			0x33			// Set Limits 10-Byte (O)

#define SCSI_SET_LIM_12			0xB3			// Set Limits 10-Byte (O)

#define SCSI_VERIFY10				0x2F			// Verify 10-Byte (O)

#define SCSI_VERIFY12				0xAF			// Verify 12-Byte (O)

#define SCSI_WRITE12				0xAA			// Write 12-Byte (O)

#define SCSI_WRT_VER10				0x2E			// Write and Verify 10-Byte (O)

#define SCSI_WRT_VER12				0xAE			// Write and Verify 12-Byte (O)

//***************************************************************************

//				  %%% Commands Unique to CD-ROM Devices %%%

//***************************************************************************

#define SCSI_PLAYAUD_10			0x45			// Play Audio 10-Byte (O)

#define SCSI_PLAYAUD_12			0xA5			// Play Audio 12-Byte 12-Byte (O)

#define SCSI_PLAYAUDMSF			0x47			// Play Audio MSF (O)

#define SCSI_PLAYA_TKIN			0x48			// Play Audio Track/Index (O)

#define SCSI_PLYTKREL10			0x49			// Play Track Relative 10-Byte (O)

#define SCSI_PLYTKREL12			0xA9			// Play Track Relative 12-Byte (O)

#define SCSI_READCDCAP				0x25			// Read CD-ROM Capacity (MANDATORY)

#define SCSI_READHEADER			0x44			// Read Header (O)

#define SCSI_SUBCHANNEL			0x42			// Read Subchannel (O)

#define SCSI_READ_TOC				0x43			// Read TOC (O)

//***************************************************************************

//				  %%% Commands Unique to Scanner Devices %%%

//***************************************************************************

#define SCSI_GETDBSTAT				0x34			// Get Data Buffer Status (O)

#define SCSI_GETWINDOW				0x25			// Get Window (O)

#define SCSI_OBJECTPOS				0x31			// Object Postion (O)

#define SCSI_SCAN					0x1B			// Scan (O)

#define SCSI_SETWINDOW				0x24			// Set Window (MANDATORY)

//***************************************************************************

//			 %%% Commands Unique to Optical Memory Devices %%%

//***************************************************************************

#define SCSI_UpdateBlk				0x3D			// Update Block (O)

//***************************************************************************

//			 %%% Commands Unique to Medium Changer Devices %%%

//***************************************************************************

#define SCSI_EXCHMEDIUM			0xA6			// Exchange Medium (O)

#define SCSI_INITELSTAT			0x07			// Initialize Element Status (O)

#define SCSI_POSTOELEM				0x2B			// Position to Element (O)

#define SCSI_REQ_VE_ADD			0xB5			// Request Volume Element Address (O)

#define SCSI_SENDVOLTAG			0xB6			// Send Volume Tag (O)

//***************************************************************************

//			  %%% Commands Unique to Communication Devices %%%

//***************************************************************************

#define SCSI_GET_MSG_6				0x08			// Get Message 6-Byte (MANDATORY)

#define SCSI_GET_MSG_10			0x28			// Get Message 10-Byte (O)

#define SCSI_GET_MSG_12			0xA8			// Get Message 12-Byte (O)

#define SCSI_SND_MSG_6				0x0A			// Send Message 6-Byte (MANDATORY)

#define SCSI_SND_MSG_10			0x2A			// Send Message 10-Byte (O)

#define SCSI_SND_MSG_12			0xAA			// Send Message 12-Byte (O)

//---------------------------------------------------------------------------

//

//					  %%% END OF SCSI COMMAND OPCODES %%%

//

//---------------------------------------------------------------------------

//***************************************************************************

//						%%% Request Sense Data Format %%%

//***************************************************************************

typedef struct {

BYTE		ErrorCode;				// Error Code (70H or 71H)

BYTE		SegmentNum;				// Number of current segment descriptor

BYTE		SenseKey;				// Sense Key(See bit definitions too)

BYTE		InfoByte0;				// Information MSB

BYTE		InfoByte1;				// Information MID

BYTE		InfoByte2;				// Information MID

BYTE		InfoByte3;				// Information LSB

BYTE		AddSenLen;				// Additional Sense Length

BYTE		ComSpecInf0;			// Command Specific Information MSB

BYTE		ComSpecInf1;			// Command Specific Information MID

BYTE		ComSpecInf2;			// Command Specific Information MID

BYTE		ComSpecInf3;			// Command Specific Information LSB

BYTE		AddSenseCode;			// Additional Sense Code

BYTE		AddSenQual;				// Additional Sense Code Qualifier

BYTE		FieldRepUCode;			// Field Replaceable Unit Code

BYTE		SenKeySpec15;			// Sense Key Specific 15th byte

BYTE		SenKeySpec16;			// Sense Key Specific 16th byte

BYTE		SenKeySpec17;			// Sense Key Specific 17th byte

BYTE		AddSenseBytes;			// Additional Sense Bytes

} SENSE_DATA_FMT;

//***************************************************************************

//						 %%% REQUEST SENSE ERROR CODE %%%

//***************************************************************************

#define SERROR_CURRENT				0x70			// Current Errors

#define SERROR_DEFERED				0x71			// Deferred Errors

//***************************************************************************

//					 %%% REQUEST SENSE BIT DEFINITIONS %%%

//***************************************************************************

#define SENSE_VALID				0x80			// Byte 0 Bit 7

#define SENSE_FILEMRK				0x80			// Byte 2 Bit 7

#define SENSE_EOM					0x40			// Byte 2 Bit 6

#define SENSE_ILI					0x20			// Byte 2 Bit 5

//***************************************************************************

//				 %%% REQUEST SENSE SENSE KEY DEFINITIONS %%%

//***************************************************************************

#define KEY_NOSENSE				0x00			// No Sense

#define KEY_RECERROR				0x01			// Recovered Error

#define KEY_NOTREADY				0x02			// Not Ready

#define KEY_MEDIUMERR				0x03			// Medium Error

#define KEY_HARDERROR				0x04			// Hardware Error

#define KEY_ILLGLREQ				0x05			// Illegal Request

#define KEY_UNITATT				0x06			// Unit Attention

#define KEY_DATAPROT				0x07			// Data Protect

#define KEY_BLANKCHK				0x08			// Blank Check

#define KEY_VENDSPEC				0x09			// Vendor Specific

#define KEY_COPYABORT				0x0A			// Copy Abort

#define KEY_EQUAL					0x0C			// Equal (Search)

#define KEY_VOLOVRFLW				0x0D			// Volume Overflow

#define KEY_MISCOMP				0x0E			// Miscompare (Search)

#define KEY_RESERVED				0x0F			// Reserved

//***************************************************************************

//				  %%% PERIPHERAL DEVICE TYPE DEFINITIONS %%%

//***************************************************************************

#define DTYPE_DASD					0x00			// Disk Device

#define DTYPE_SEQD					0x01			// Tape Device

#define DTYPE_PRNT					0x02			// Printer

#define DTYPE_PROC					0x03			// Processor

#define DTYPE_WORM					0x04			// Write-once read-multiple

#define DTYPE_CROM					0x05			// CD-ROM device

#define DTYPE_SCAN					0x06			// Scanner device

#define DTYPE_OPTI					0x07			// Optical memory device

#define DTYPE_JUKE					0x08			// Medium Changer device

#define DTYPE_COMM					0x09			// Communications device

#define DTYPE_RESL					0x0A			// Reserved (low)

#define DTYPE_RESH					0x1E			// Reserved (high)

#define DTYPE_UNKNOWN				0x1F			// Unknown or no device type

//***************************************************************************

//				  %%% ANSI APPROVED VERSION DEFINITIONS %%%

//***************************************************************************

#define ANSI_MAYBE					0x0		// Device may or may not be ANSI approved stand

#define ANSI_SCSI1					0x1		// Device complies to ANSI X3.131-1986 (SCSI-1)

#define ANSI_SCSI2					0x2		// Device complies to SCSI-2

#define ANSI_RESLO					0x3		// Reserved (low)

#define ANSI_RESHI					0x7		// Reserved (high)