LOBAT, a more reliable low-batteries test for programmers, by Joe Horn

CAUTION!  Load the version for your machine only!

LOBAT.SX --> HP48 S or SX only.

LOBAT.GX --> HP48 G or GX only.

The 'ALERT' program by Preston Brown on Goodies Disk #1 checks the

status of the "alert annunciator" (the low-battery and past-due alarm

indicator).  Although it is reliable for testing the annunciator, it

is not a reliable method for testing the batteries, because the alert

annunciator is only updated occasionally.  Merely testing the

annunciator itself does not actually perform the internal battery

test.  LOBAT does.

The System-RPL version is short and fast: 17.5 bytes, 4 milliseconds.

INSTRUCTIONS:

INPUT: None.

OUTPUT: 0  --> all 5 batteries are okay

1  --> main batteries (AAA) are low

2  --> Slot 2 RAM card battery is low

4  --> Slot 1 RAM card battery is low

Multiple low-battery indications are added together; for example, if

LOBAT returns 5, that would mean that the main batts *and* slot 1 are

both low (because 5=1+4).  Thus LOBAT can return any number from 0 to

7, with 1 through 7 indicating low batt(s).

'LOBAT' can therefore be used as a true/false test in programs.  For

example, DO ... UNTIL LOBAT END would exit a loop as soon as a low

battery condition is detected.

<< DO IF LOBAT THEN 1400 .01 BEEP END UNTIL 0 END >> is silent for

healthy batteries, but it begins to chirp when the main battery

voltage gets down to about 3.2 volts, and soon begins to sound like a

Geiger counter, and finally emits a non-stop noise when the voltage

drops below about 3.15 volts.  Values vary slightly from machine to

machine.

LOBAT ignores empty card slots.  If a card slot has no RAM card

installed, LOBAT will NOT give a low-battery indication for it.

Note regarding I/O:

The LOBAT program checks the low-batt condition *now*, not the

low-batt condition that *will* happen *if* you start doing I/O.  When

the batteries are just barely above low-batt level, the LOBAT program

returns a 0 (as it should), but then if you attempt I/O (which causes

an immediate, heavy power drain) the batteries quickly fall below the

low-batt threshold, and I/O is aborted.  Often as not, the battery

level just as quickly recovers (alkalines recover VERY quickly), and

the low-batt condition disappears.  This creates the illusion that I/O

is detecting a low-batt condition that doesn't exist, or conversely

that LOBAT is failing to detect a low-batt condition that does exist.

The low-batt condition *did* exist, but only at the moment of

attempted I/O.  It's a threshold dilemma.