CLK.DOC File

(c) Hewlett-Packard Company, 1992.

Overview

--------

The CLK application keeps the system clock as accurate as possible by

synchronizing the clock with a time standard, and using the accumulated

adjustments with the elapsed time to compute the necessary correction

amount/rate.  It is especially useful for applications like navigation that

require extremely accurate times.

Adjustments are made relative to the nearest number of 1/2 hour time-zone

changes. All changes that are multiples of 1/2 hour are assumed time-zone

changes (and ignored in error correction computation); other changes are

assumed error corrections.  For example, a change of +1:35 is assumed to be

+1:30 time-zone change (ignored) and +:05 error correction (the clock is 5

minutes slow).  A change of +1:25 is assumed to be +1:30 time-zone and -:05

error (the clock is 5 minutes fast).

Installation

------------

Transfer CLK into your HP 48 HOME directory.  (Don't use the HP 48 built-in

TIME SET/ADJST operations while this application is active.)

Operation

---------

1.  Press [VAR]|CLK| to access the application.

2.  Enter a list containing the minimum interrupt rate and the maximum

clock deviation allowed, press |C.RST|.  (This initializes the

application.)

3.  If necessary, enter a list containing the current time and date, press

|C.SET|.

4.  Enter the desired adjustment, press |C.ADJ|.

5.  Repeat steps 4 and 5 as needed.

6.  Press |C.XCT| when the clock is synchronized.

7.  Wait a few weeks and then repeat steps 4, 5, 6, and 7.

8.  Press |C.STS| to review the status information.

9.  Press |C.SSP| to suspend the correction alarm cycle temporarily; press

|C.CNT| to continue the suspended correction alarm.

10. Repeat step 3 to cancel the correction alarm and reinitialize all CLK

variables.

---------------------------------------------------------------------------

Note:  If you abort the program, flags that were changed by the program

will remain changed.

---------------------------------------------------------------------------

User Menu Keys

--------------

The following five programs make up the user interface to this package.

Each will give an error message if the expected input is not found. The

error message attempts to indicate the form of the input expected. Time

input should be in the form HH.MMSSs, and date input should be MM.DDYYYY.

|C.SET|   Takes a level-1 list containing the current time and date.  The

clock is set to this time/date.

|C.ADJ|   Takes from level 1 the amount of adjustment to be applied to the

current time.  A positive amount increases the time and a

negative amount decreases it.

|C.XCT|   Takes nothing from the stack; tells the correction package that

the clock is now exactly synchronized with the time standard.  If

this is the first time it has been invoked, then a time base is

activated and all error corrections from this point on will be

accumulated.  If this is the second or subsequent invocation,

then the time base is extended to this point and the necessary

calculations are done to set up the clock correction package.  An

alarm is scheduled and the correction package will make the

needed clock corrections from now on.  Subsequent invocations

will refine the accuracy of the corrections.

|C.RST|   Takes a level-1 list containing the minimum interrupt rate and

the maximum clock deviation allowed (both in "time" format).  The

correction package is reset to a virgin state--the time base is

deactivated, the alarm is removed, and further error corrections

will not be accumulated (the accumulators are cleared).  All

variables are reinitialized and the directory is reordered.

Maximum clock deviation specifies the maximum acceptable error

for the clock. If the absolute value of the given maximum

deviation is less than .01 second, then it is set to 1 second.

Minimum interrupt rate specifies the minimum amount of time

required between clock adjustments.  If the absolute value of the

given minimum interrupt rate is less than 1 second, then it is

set to 1 minute.  If the clock cannot be adjusted to accurate

time with the given maximum deviation without using alarms at a

more frequent rate than allowed by the minimum interrupt rate,

the correction amount will be increased as needed to ensure that

alarms do not occur more often.  Consequently, the clock may not

be as accurate as specified by the maximum deviation.  This

ensures that alarms are not occurring so often that the HP 48 has

no time to do anything else (see the "Note" at the end of the

documentation).

|C.STS|   Takes no input from level 1; provides 3 pages of status

information about the correction package.

All pages have this header:

Correction status     -N-    (Where N = 1, 2, or 3)

All pages have this foot:

prev:{up-arrow}  exit:{down arrow}  next:{right arrow}

The first page of status information looks like this:

Exact initiated(*):

9.1455  10.301989

Deviation:   0.000025

Min rate:    1.0000

The first two lines show the time and date of the first exact

synchronization.  If the display shows "none," an exact has never

been done.  If there is an (*), then 2 or more exacts have been

done and the package is fully activated.  If there is no (*),

only one exact has been done (errors are being accumulated but no

corrections are being done yet).  The third line shows user

specified maximum clock deviation (1/4 second in example).  The

fourth line shows the user specified minimum interrupt rate (1

hour in example).

The second page contains adjustment data:

Timezones:    0.0000

Errors:       0.001890

Timebase:  1718.3909

The first line shows the total accumulated time-zone changes.

The second line shows the total accumulated error corrections

(18.9 seconds in example).  The third line shows the total time

included in the time base (1718 hours, 39 minutes, and 9 seconds

in example).

The third page contains correction data:

Next correction:

11.1644  1.101990

Repeat:     22.4520

Adjust:      0.000025

The first line may be either "Next correction:" (the next

correction time is displayed) or "Adjust suspended:" (the time

that the corrections were suspended is displayed).  The third

line shows the repeat interval of the alarm (22 hours, 45

minutes, and 20 seconds in example).  The fourth line shows the

correction adjustment amount (+1/4 second in example).

If a status is displayed for 1 minute with no keys pressed, it

times out and exits.

|C.SSP|   Suspends the correction alarm (if corrections are active). The

status report will indicate the time of the suspension. If the

|C.SET|, |C.ADJ|, or |C.XCT| keys are used while the correction

alarm is suspended, the results may not be correct. (|C.SSP| is

found on page 2 of the menu.)

|C.CNT|   Resumes (continues) the correction alarm cycle if corrections are

active. Any missed corrections will be made and the next

correction alarm will be set up. (|C.CNT| is found on page 2 of

the menu.)

Application Menu Keys

---------------------

The |C.TLS| menu key contains subroutines and variables used internally by

the application. You do not need to use them to run the application.

---------------------------------------------------------------------------

Note:  The alarm should work regardless of the directory path when the

alarm occurs, and it should leave things as they were when it

happened.  However, if the HP 48 is off when the alarm occurs, it

will be turned on and left on until the HP 48 times out. If alarms

occur at a rate less than the time-out period, the HP 48 will

always be on. If anything is being done when the alarm occurs, the

alarm will either not process immediately (when a program is

running, the alarm doesn't process until the program terminates),

or it will abort what is happening (edits will be terminated as if

the [ATTN] key had been pressed).

---------------------------------------------------------------------------

V. 1.0 04/26/92 04:09 PM